EEPROM DEBUGGING

### Let's test the EEPROM independently first

Don't debug the whole tank project yet.

Temporarily make a simple test:

uint8_t testWrite = 25;

uint8_t testRead = 0;

EEPROM_WriteByte(0x0000, testWrite);

HAL_Delay(10);

testRead = EEPROM_ReadByte(0x0000);

Put a breakpoint after:

testRead = EEPROM_ReadByte(0x0000);

You should get:

testWrite = 25

testRead  = 25

If you get:

testRead = 0xFF

then the EEPROM driver/wiring is the problem, not your menu code.

# 1. Your EEPROM address 0xA0 is correct for HAL

You have:

#define EEPROM_ADDR 0xA0

For AT24C32, the normal 7-bit I²C address is:

0x50

But STM32 HAL functions expect the address in the 8-bit shifted format:

0x50 << 1 = 0xA0

Therefore:

#define EEPROM_ADDR 0xA0

is correct. Don't change it to 0x50 yet.

# 2. Your write function isn't checking whether the write succeeded

Currently:

HAL_I2C_Master_Transmit(&hi2c1,

EEPROM_ADDR,

buffer,

3,

HAL_MAX_DELAY);

HAL_Delay(5);

You're ignoring the return value.

Let's temporarily change it to:

HAL_StatusTypeDef status;

status = HAL_I2C_Master_Transmit(&hi2c1,

EEPROM_ADDR,

buffer,

3,

HAL_MAX_DELAY);

if (status != HAL_OK)

{

Error_Handler();

}

HAL_Delay(5);

Now if the STM32 cannot communicate with the EEPROM, you'll know immediately because it will enter Error_Handler().

# 5. Let's isolate the EEPROM before debugging your menu

This is what I want you to do now.

Temporarily put this in main.c, after:

MX_I2C1_Init();

inside USER CODE BEGIN 2:

uint8_t testRead;

EEPROM_WriteByte(0x0000, 25);

HAL_Delay(10);

testRead = EEPROM_ReadByte(0x0000);

Then put a breakpoint here:

testRead = EEPROM_ReadByte(0x0000);

Step over it and look at:

testRead

### Expected:

testRead = 25

If you get:

testRead = 25

🎯 EEPROM hardware + driver are working.

Then we can go back to your menu code.

# 6. If testRead is 0xFF

Then don't touch your menu code.

Check the hardware.

Your AT24C32 should be connected approximately like:

AT24C32                  STM32

--------------------------------

VCC       -------------> 3.3V

GND       -------------> GND

SCL       -------------> I2C1_SCL

SDA       -------------> I2C1_SDA

A0        -------------> GND

A1        -------------> GND

A2        -------------> GND

WP        -------------> GND

### Very important: WP

Check the WP (Write Protect) pin.

If:

WP = HIGH

the EEPROM can prevent writes.

For our project, connect:

WP → GND

# 7. Also check whether your module has pull-ups

I²C needs pull-up resistors:

3.3V

│

[4.7kΩ]

│

SDA ───────────── STM32 SDA

3.3V

│

[4.7kΩ]

│

SCL ───────────── STM32 SCL

If you're using an AT24C32 module, it may already contain these resistors.

If you're using the bare IC, you need to provide them.

Step-by-step debugging is exactly what I recommend here. We want to trace:

main.c → EEPROM_WriteByte() → HAL_I2C_Master_Transmit() → status → Error_Handler()

### 1. Put a breakpoint here

In eeprom.c:

status = HAL_I2C_Master_Transmit(&hi2c1,

EEPROM_ADDR,

buffer,

3,

HAL_MAX_DELAY);

Put a breakpoint on this line.

Then start debugging.

### 2. When execution stops there

Before executing the line, inspect:

EEPROM_ADDR

buffer[0]

buffer[1]

buffer[2]

For your test:

EEPROM_WriteByte(0x0000, 25);

you should see:

EEPROM_ADDR = 0xA0

buffer[0]   = 0x00

buffer[1]   = 0x00

buffer[2]   = 0x19

because 25 decimal = 0x19.

### 3. Press F6 / Step Over

Execute:

status = HAL_I2C_Master_Transmit(...);

Now look at:

status

The important possibilities are:

HAL_OK

HAL_ERROR

HAL_BUSY

HAL_TIMEOUT

If it is:

HAL_OK

then the STM32 successfully communicated with the EEPROM at the HAL level.

If it is:

HAL_ERROR

then we have an I²C communication problem.

### 4. If you get HAL_ERROR

The next line is:

if (status != HAL_OK)

{

Error_Handler();

}

Press F6 (Step Over).

You should then enter:

void Error_Handler(void)

{

__disable_irq();

while (1)

{

}

}

The debugger will stop inside the infinite:

while (1)

That proves:

HAL_I2C_Master_Transmit() failed.

### 5. If you get HAL_OK

Then do not enter Error_Handler().

Continue with:

HAL_Delay(5);

Then return to:

main.c

and execute:

testRead = EEPROM_ReadByte(0x0000);

Put another breakpoint here:

testRead = EEPROM_ReadByte(0x0000);

After stepping over it, inspect:

testRead

### Your complete debugging path

main.c

│

▼

EEPROM_WriteByte(0x0000, 25)

│

▼

eeprom.c

│

▼

HAL_I2C_Master_Transmit()

│

├── HAL_OK ────────► continue

│

└── HAL_ERROR ────► Error_Handler()

│

▼

while(1)

Then:

EEPROM_WriteByte()

↓

EEPROM_ReadByte()

↓

testRead

↓

Should be 25