Now we'll write the HC-SR04 driver ourselves

# Step 1: Project Structure

Create two new files:

Core/Src/ultrasonic.c

Core/Inc/ultrasonic.h

This keeps the sensor driver separate from main.c.

## Step 1: Open Project Explorer

On the left side of STM32CubeIDE, you'll see something like:

Industrial_Tank_Level

│

├── Core

│   ├── Inc

│   └── Src

├── Drivers

├── Debug

└── ...

## Step 2: Create ultrasonic.c

- Right-click Core → Src

- Select New → Source File

- Enter:

ultrasonic.c

- Click Finish

Your project will look like:

Core

│

├── Inc

│

└── Src

├── main.c

├── stm32f0xx_it.c

├── ultrasonic.c   ← New

## Step 3: Create ultrasonic.h

- Right-click Core → Inc

- Select New → Header File

- Enter:

ultrasonic.h

- Click Finish

Now it should look like:

Core

│

├── Inc

│   ├── main.h

│   ├── ultrasonic.h   ← New

│

└── Src

├── main.c

├── ultrasonic.c   ← New

## Why keep them in Core/Inc and Core/Src?

STM32CubeIDE automatically includes the Core/Inc folder in the compiler's include path. This means you can simply write:

#include "ultrasonic.h"

without changing any project settings.

For small and medium-sized projects, this is the standard approach.

## Later (Professional Project Structure)

As the project grows, we can organize drivers like this:

Industrial_Tank_Level

│

├── Core

│

├── Drivers

│

├── Modules

│   ├── Ultrasonic

│   │   ├── ultrasonic.c

│   │   └── ultrasonic.h

│   ├── LCD

│   ├── EEPROM

│   └── Alarm

This is a more scalable structure used in industry, but for now we'll keep everything in Core/Inc and Core/Src to avoid unnecessary configuration.

# Step 2: ultrasonic.h

#ifndef ULTRASONIC_H

#define ULTRASONIC_H

#include "main.h"

void HCSR04_Trigger(void);

float HCSR04_GetDistance(void);

#endif

## Why use a header file?

The header file contains function declarations. This allows any source file (such as main.c) to call these functions without knowing their implementation.

# Where should you write this?

Create the file:

Core

└── Inc

└── ultrasonic.h

Open ultrasonic.h and write the following:

#ifndef ULTRASONIC_H

#define ULTRASONIC_H

#include "main.h"

void HCSR04_Trigger(void);

float HCSR04_GetDistance(void);

#endif

This entire code belongs only in ultrasonic.h, not in main.c.

# Now let's understand every line.

## Line 1

#ifndef ULTRASONIC_H

#ifndef means:

If ULTRASONIC_H is NOT defined...

Think of it as asking:

Has this file already been included?

No?

↓

Then include it.

## Line 2

#define ULTRASONIC_H

Now the compiler remembers:

ULTRASONIC_H

is defined.

If another file later tries to include ultrasonic.h again, the compiler skips it.

## Why is this needed?

Imagine:

main.c

|

+------> ultrasonic.h

|

+------> lcd.h

|

+------> ultrasonic.h

Without #ifndef, the compiler reads ultrasonic.h twice.

That causes errors like:

multiple definition

redefinition

The include guard prevents that.

## Line 3

#include "main.h"

This tells the compiler:

Include everything from main.h.

Why?

Because main.h already contains useful definitions such as:

GPIO_PIN_0

GPIOA

HAL_GPIO_WritePin()

Without including main.h, the compiler won't recognize those symbols.

## Function Declarations

void HCSR04_Trigger(void);

This is not the function itself.

It only tells the compiler:

There is a function named HCSR04_Trigger.

The actual code will be written in:

ultrasonic.c

### Example

In ultrasonic.h:

void LED_ON(void);

In ultrasonic.c:

void LED_ON(void)

{

HAL_GPIO_WritePin(GPIOA, GPIO_PIN_5, GPIO_PIN_SET);

}

Header file = declaration

Source file = implementation

## Next Function

float HCSR04_GetDistance(void);

This tells the compiler:

There is a function

HCSR04_GetDistance()

which returns a float.

Example:

float distance;

distance = HCSR04_GetDistance();

If the measured distance is:

35.6 cm

the function returns:

35.6

## Last Line

#endif

This closes the #ifndef.

Think of it like:

#ifndef

...

#endif

Everything between them is protected from being included more than once.

# Summary

ultrasonic.h

Contains

✔ Function declarations

✔ Macros

✔ Include files

✔ Structure declarations

NO function code

ultrasonic.c

Contains

✔ Actual function implementations

✔ Algorithm

✔ Sensor driver code

# Generate the Trigger Pulse

In ultrasonic.c:

#include "ultrasonic.h"

extern TIM_HandleTypeDef htim6;

void HCSR04_Trigger(void)

{

HAL_GPIO_WritePin(TRIG_PORT, TRIG_PIN, GPIO_PIN_RESET);

delay_us(2);

HAL_GPIO_WritePin(TRIG_PORT, TRIG_PIN, GPIO_PIN_SET);

delay_us(10);

HAL_GPIO_WritePin(TRIG_PORT, TRIG_PIN, GPIO_PIN_RESET);

}

## Why these three steps?

### Step 1

HAL_GPIO_WritePin(...RESET);

Makes sure the Trigger pin starts LOW.

### Step 2

delay_us(2);

Keeps it LOW for a short time to ensure a clean pulse.

### Step 3

HAL_GPIO_WritePin(...SET);

delay_us(10);

Creates the required 10 µs HIGH pulse.

The HC-SR04 datasheet specifies that the trigger input must be HIGH for at least 10 µs.

### Step 4

HAL_GPIO_WritePin(...RESET);

Ends the trigger pulse.

# What Does the Trigger Look Like?

MCU Trigger

__________|‾‾‾‾‾‾‾‾‾|________

10 µs

After this pulse, the HC-SR04 sends ultrasonic waves and then drives the Echo pin HIGH for a duration proportional to the measured distance.

# The line

extern TIM_HandleTypeDef htim6;

has three parts:

extern      TIM_HandleTypeDef      htim6

|                |                |

Keyword         Data Type       Variable Name

Let's understand each one.

# Step 1: What is TIM_HandleTypeDef?

When CubeMX generates code, it creates a variable like this in main.c:

TIM_HandleTypeDef htim6;

Here,

TIM_HandleTypeDef

is a structure (struct) defined by the HAL library.

Think of it like a file that stores all information about TIM6.

Example (simplified):

typedef struct

{

TIM_TypeDef *Instance;

TIM_Base_InitTypeDef Init;

HAL_TIM_StateTypeDef State;

} TIM_HandleTypeDef;

This structure stores things like:

- Which timer (TIM6)

- Prescaler

- Period

- Timer state

- Other configuration

# Step 2: What is htim6?

This is simply the name of the variable.

Just like:

int age;

where

int

is the type and

age

is the variable.

Similarly,

TIM_HandleTypeDef htim6;

means:

Type                 Variable

TIM_HandleTypeDef    htim6

# Where is htim6 created?

Open your main.c.

Near the top you'll find something like:

TIM_HandleTypeDef htim2;

TIM_HandleTypeDef htim6;

I2C_HandleTypeDef hi2c1;

UART_HandleTypeDef huart1;

These are global variables.

# Problem

Suppose we write:

### main.c

TIM_HandleTypeDef htim6;

Now go to:

### ultrasonic.c

void delay_us(uint16_t us)

{

__HAL_TIM_SET_COUNTER(&htim6,0);

}

Will this compile?

❌ No.

Why?

Because htim6 belongs to main.c.

ultrasonic.c has no idea that this variable exists.

# Solution: extern

We tell the compiler:

"Don't create a new variable. A variable named htim6 already exists somewhere else."

That's exactly what:

extern TIM_HandleTypeDef htim6;

means.

# Visual Explanation

### main.c

TIM_HandleTypeDef htim6;

Memory:

RAM

+------------------+

|      htim6       |

+------------------+

Now another file wants to use it.

### ultrasonic.c

extern TIM_HandleTypeDef htim6;

This doesn't create another copy.

Instead:

ultrasonic.c

|

|

▼

+------------------+

|      htim6       |

+------------------+

(main.c)

Both files refer to the same object in memory.

# What if we remove extern?

Suppose we write:

TIM_HandleTypeDef htim6;

inside ultrasonic.c.

Now we have:

main.c

htim6

and

ultrasonic.c

htim6

Two variables with the same name.

The linker will report an error like:

multiple definition of htim6

# Why extern doesn't allocate memory

Compare these two statements:

### Creates memory

int x;

Memory:

RAM

x

### Doesn't create memory

extern int x;

Memory:

No memory allocated

Just tells compiler:

"x exists elsewhere."

# Real-Life Analogy

Imagine a classroom.

There is one whiteboard.

Teacher (main.c):

"There is a whiteboard."

Student (ultrasonic.c):

"extern whiteboard"

The student is saying:

"I know the whiteboard already exists. I just want to use it."

The student is not bringing another whiteboard.

# Why do we pass &htim6?

You have seen:

__HAL_TIM_SET_COUNTER(&htim6,0);

Notice the &.

This means:

Address of htim6

The HAL function needs access to the timer handle so it can modify its fields.

Without &:

htim6

You pass the whole structure.

With &:

&htim6

You pass its memory address (a pointer), allowing the function to work directly with the original handle.

# Interview Question

Interviewer:

Why do we write extern TIM_HandleTypeDef htim6;?

Answer:

htim6 is defined globally in main.c by STM32CubeMX. To use the same timer handle in another source file such as ultrasonic.c, we declare it with extern. The extern keyword tells the compiler that the variable is defined in another translation unit and avoids creating a second instance, preventing multiple-definition linker errors.

Excellent. Now we'll make the code professional. Right now, Distance is a global variable that any file can change. In embedded firmware, that's considered poor design.

Let's improve it.

# Lesson 7: Build a Proper Driver API

Instead of doing this in main.c:

Distance = Difference / 58.0f;

printf("%f", Distance);

we want:

float distance;

distance = HCSR04_GetDistance();

Notice that main.c doesn't know how the distance is measured. It only asks the driver for the latest value.

This is called abstraction.

# Current Situation

main.c

│

│ directly accesses

▼

Distance (Global Variable)

Any file can accidentally do:

Distance = 1000;

Now your measurement is corrupted.

# Better Design

main.c

│

▼

HCSR04_GetDistance()

│

▼

ultrasonic.c

│

▼

Distance (Private)

Now only the driver can modify Distance.

# Step 1: Make Distance private

In ultrasonic.c, change:

float Distance = 0;

to:

static float Distance = 0;

## What does static mean here?

Since this variable is outside any function, static means:

This variable can only be used inside ultrasonic.c.

It still keeps its value between function calls, but it is no longer visible from other source files.

# Before

main.c

│

▼

Distance

Possible.

# After

main.c

Cannot access Distance ❌

Only:

ultrasonic.c

can access it.

# Step 2: Implement HCSR04_GetDistance()

In ultrasonic.c:

float HCSR04_GetDistance(void)

{

return Distance;

}

Very simple.

This function just returns the latest measured value.

# Step 3: Use it in main.c

Instead of:

printf("%f", Distance);

write:

float distance;

distance = HCSR04_GetDistance();

Now main.c doesn't know where the value came from.

That's exactly what we want.

# Why is this better?

Imagine next month you replace the HC-SR04 with another sensor.

Only ultrasonic.c changes.

main.c still does:

distance = HCSR04_GetDistance();

No changes needed.

This is one of the main goals of modular programming.

# Interview Question

Interviewer: Why did you make Distance static?

Good Answer:

Distance is an internal implementation detail of the ultrasonic driver. Declaring it static restricts its visibility to ultrasonic.c, preventing accidental modification from other modules and improving encapsulation.

# Lesson 8: Triggering a Measurement

Now we have everything needed to measure distance.

In the while(1) loop of main.c:

while (1)

{

HCSR04_Trigger();

HAL_Delay(100);

float distance = HCSR04_GetDistance();

}

Let's understand the sequence:

### Step 1

HCSR04_Trigger();

Sends the 10 µs trigger pulse.

### Step 2

The HC-SR04 responds by generating the Echo pulse.

TIM2 automatically captures:

- Rising edge

- Falling edge

The interrupt calculates the distance and stores it in Distance.

You never call the callback directly.

### Step 3

HAL_Delay(100);

Wait 100 ms before the next measurement.

The HC-SR04 datasheet recommends leaving enough time between trigger pulses. A 100 ms interval is a safe choice for this project.

### Step 4

float distance = HCSR04_GetDistance();

Retrieve the latest measured distance.

# Complete Measurement Flow

while(1)

│

▼

HCSR04_Trigger()

│

▼

HC-SR04 sends Echo pulse

│

▼

TIM2 captures Rising Edge

│

▼

Interrupt

│

▼

TIM2 captures Falling Edge

│

▼

Interrupt

│

▼

Distance calculated

│

▼

HCSR04_GetDistance()