# Microsecond Delay

Earlier, you already created a delay function using TIM6.

void delay_us(uint16_t us)

{

__HAL_TIM_SET_COUNTER(&htim6,0);

while(__HAL_TIM_GET_COUNTER(&htim6) < us);

}

### How does this work?

Suppose TIM6 runs at 1 MHz.

That means:

Counter

0

1

2

3

4

...

Each increment equals:

1 µs

If you call:

delay_us(10);

The loop waits until the counter reaches 10.

Result:

10 µs delay

# Why Use TIM6?

Instead of:

for(i=0;i<100;i++);

we use a hardware timer because:

- It is independent of compiler optimization.

- It provides repeatable timing.

- It is much more accurate.

# Interview Question

Q: Why not use HAL_Delay(1)?

Answer:

HAL_Delay() has 1 ms resolution, while the HC-SR04 requires a 10 µs trigger pulse. A hardware timer is needed to generate delays in microseconds.

## Create a separate delay.c and delay.h

As your project grows, you'll need microsecond delays in more than one module. So create:

Core

├── Inc

│   └── delay.h

└── Src

└── delay.c

delay.h

#ifndef DELAY_H

#define DELAY_H

#include "main.h"

void delay_us(uint16_t us);

#endif

delay.c

#include "delay.h"

extern TIM_HandleTypeDef htim6;

void delay_us(uint16_t us)

{

__HAL_TIM_SET_COUNTER(&htim6, 0);

while (__HAL_TIM_GET_COUNTER(&htim6) < us);

}

Now any file can use it by writing:

#include "delay.h"

This is the approach I recommend for your project.

## Option 2 (Temporary): Put it in main.c

If you're just testing, you can put the function inside main.c.

Place it in the USER CODE section, for example:

/* USER CODE BEGIN 0 */

void delay_us(uint16_t us)

{

__HAL_TIM_SET_COUNTER(&htim6,0);

while(__HAL_TIM_GET_COUNTER(&htim6) < us);

}

/* USER CODE END 0 */

But later, we'll move it to delay.c.

# Which approach should you use?

Since you're building this project for embedded interviews, I recommend this structure:

Industrial_Tank_Level

│

├── Core

│   ├── Inc

│   │   ├── ultrasonic.h

│   │   ├── delay.h

│   │   └── main.h

│   │

│   └── Src

│       ├── main.c

│       ├── ultrasonic.c

│       └── delay.c

This looks much more professional than putting everything in main.c.

- ✅ Prescaler = 48 - 1 = 47

- ✅ Counter Mode = Up

- ✅ Counter Period = 65535

- ✅ Auto Reload Preload = Disable

Since your timer clock is 48 MHz:

Timer Frequency=48 MHz47+1=1 MHz\text{Timer Frequency}=\frac{48\,MHz}{47+1}=1\,MHzTimer Frequency=47+148MHz​=1MHz

That means:

- 1 timer count = 1 µs

So your delay_us() function will work correctly.

## One More Important Step

The timer must actually start before you use delay_us().

In main.c, after all peripheral initialization, make sure you have:

/* USER CODE BEGIN 2 */

HAL_TIM_Base_Start(&htim6);

/* USER CODE END 2 */

If you don't start TIM6, the counter will never increment, and this line:

while(__HAL_TIM_GET_COUNTER(&htim6) < us);

will wait forever.

## Now let's create the delay module

### 1. Create delay.h

Location:

Core

└── Inc

└── delay.h

Contents:

#ifndef DELAY_H

#define DELAY_H

#include "main.h"

void delay_us(uint16_t us);

#endif

### 2. Create delay.c

Location:

Core

└── Src

└── delay.c

Contents:

#include "delay.h"

extern TIM_HandleTypeDef htim6;

void delay_us(uint16_t us)

{

__HAL_TIM_SET_COUNTER(&htim6, 0);

while (__HAL_TIM_GET_COUNTER(&htim6) < us);

}

### 3. Use it in ultrasonic.c

At the top of ultrasonic.c:

#include "ultrasonic.h"

#include "delay.h"

Now you can simply call:

delay_us(10);

## Why did we make a separate delay.c?

Imagine later you add:

- DHT11

- DS18B20

- WS2812 LEDs

- OneWire EEPROM

All of them need microsecond delays.

Instead of copying the same function into every file:

❌ Bad:

main.c

delay_us()

dht11.c

delay_us()

ultrasonic.c

delay_us()

We write it once:

delay.c

↓

All modules use it

This is called code reuse, and it's standard practice in embedded software development.