# Understanding TIM2 Before Configuration

Many people copy timer settings from YouTube. We are going to understand why each setting is chosen.

# What is a Timer?

A timer is simply a hardware counter.

Think of it like this:

0 → 1 → 2 → 3 → 4 → 5 → ...

The timer increments automatically based on the timer clock.

Unlike a software variable, the CPU doesn't need to increment it.

# How Fast Does It Count?

Suppose your timer clock is 48 MHz.

That means:

48,000,000 counts every second

or

1 count every

1 / 48,000,000

= 20.83 ns

So without a prescaler:

| Timer Count | Time |
| --- | --- |
| 1 | 20.83 ns |
| 100 | 2.08 µs |
| 1000 | 20.8 µs |

This is very accurate, but not convenient for ultrasonic calculations.

# The Problem

The HC-SR04 Echo pulse is typically:

100 µs

500 µs

2000 µs

10000 µs

If the timer counts every 20.8 ns, the captured values become huge.

Example:

2000 µs

↓

96000 counts

Hard to interpret.

# The Solution: Prescaler

The prescaler slows the timer.

Formula:

Timer Frequency=Timer ClockPSC+1\boxed{\text{Timer Frequency}=\frac{\text{Timer Clock}}{\text{PSC}+1}}Timer Frequency=PSC+1Timer Clock​​

Where:

- Timer Clock = 48 MHz

- PSC = Prescaler register

# Example 1

PSC = 0

48 MHz / (0+1)

=48 MHz

One count:

20.8 ns

# Example 2

PSC = 47

48 MHz /48

=1 MHz

One count becomes:

1 µs

This is exactly what we want.

Now:

Timer Count = Time in µs

# Why 1 MHz is Perfect

Suppose Echo pulse:

1982 µs

Timer capture becomes:

1982

Distance:

distance = 1982 / 58.0;

Very simple.

No complicated conversions.

# Timer Configuration

For our project:

| Parameter | Value |
| --- | --- |
| Timer Clock | 48 MHz |
| Prescaler | 47 |
| Timer Frequency | 1 MHz |
| Resolution | 1 µs/count |

# Counter Mode

Options:

Up

Down

Center Aligned

We select:

Up

Meaning:

0

1

2

3

4

5

...

# Auto Reload Register (ARR)

ARR decides when the timer resets.

Suppose:

ARR = 100

Timer:

0

1

2

...

100

↓

0

For our project:

65535

Maximum value of a 16-bit timer.

# Why 65535?

Maximum measurable time:

65535 µs

=

65.5 ms

Maximum HC-SR04 Echo:

Approximately 38 ms.

So:

38 ms

<

65.5 ms

No overflow occurs.

# Input Capture

Instead of reading GPIO continuously:

CPU

while(GPIO==1);

The timer does this:

Echo Rising Edge

↓

Capture Counter

↓

Echo Falling Edge

↓

Capture Counter

The hardware stores the counter values automatically.

# Capture Sequence

Suppose timer:

0

1

2

3

...

1500

Echo goes HIGH.

Timer stores:

1500

Echo becomes LOW:

3482

Timer stores:

3482

Pulse width:

3482-1500

=

1982

Distance:

1982/58

=

34 cm

# Final TIM2 Settings

In CubeMX:

| Parameter | Value |
| --- | --- |
| Clock Source | Internal Clock |
| Prescaler | 47 |
| Counter Mode | Up |
| Period (ARR) | 65535 |
| Channel 2 | Input Capture Direct Mode |
| Polarity | Rising Edge (we'll switch to Falling Edge in software after the first capture) |
| Filter | 0 |

# Timer Input Capture (Internal Working)

## What is Input Capture?

Input Capture is a hardware feature that records the timer count automatically when an event (edge) occurs on an input pin.

Instead of the CPU constantly checking the pin:

while(HAL_GPIO_ReadPin(GPIOA, GPIO_PIN_1));

the timer hardware does the job.

# Internal Block Diagram

PA1 (Echo)

│

▼

Input Capture Unit

│

▼

CCR2 Register

│

▼

CPU Reads Value

# What is CCR2?

Since we're using TIM2 Channel 2, the capture value is stored in:

TIM2->CCR2

CCR means:

Capture Compare Register

Channel mapping:

| Channel | Register |
| --- | --- |
| CH1 | CCR1 |
| CH2 | CCR2 |
| CH3 | CCR3 |
| CH4 | CCR4 |

# Example

Assume:

- Timer frequency = 1 MHz

- Timer counts every 1 µs

The timer starts counting:

Count

0

1

2

3

4

5

...

1000

1001

1002

...

# Rising Edge

Echo becomes HIGH.

Echo

_______|‾‾‾‾‾‾‾

↑

Suppose the timer count is:

1520

The hardware immediately stores:

TIM2->CCR2 = 1520

The CPU did nothing.

# Falling Edge

Echo becomes LOW.

Echo

_______|‾‾‾‾‾|____

↑

Timer count:

3510

Hardware stores:

TIM2->CCR2 = 3510

# Pulse Width

Pulse Width

3510

-

1520

=

1990 µs

Distance:

1990 / 58

=

34.3 cm

# Why Do We Change Edge Polarity?

Initially:

Capture Rising Edge

We want:

Start Time

After the first interrupt:

Change to:

Capture Falling Edge

Now the next interrupt gives:

End Time

Then:

Pulse Width

=

End

-

Start

Finally:

Switch back to:

Rising Edge

and wait for the next measurement.

# Complete Sequence

Trigger Pulse

│

▼

HC-SR04 Sends Ultrasound

│

▼

Echo Goes HIGH

│

▼

Capture Rising Edge

│

▼

Store Start Time

│

▼

Echo Goes LOW

│

▼

Capture Falling Edge

│

▼

Store End Time

│

▼

End - Start

│

▼

Distance

# Why Interrupts?

Without interrupts:

while(GPIOA->IDR & GPIO_PIN_1);

The CPU waits and cannot perform other work.

With Input Capture Interrupt:

Timer Hardware

↓

Edge Detected

↓

Interrupt

↓

ISR Executes

↓

Main Program Continues

This is far more efficient.

# HAL Functions You'll Use

Enable Input Capture:

HAL_TIM_IC_Start_IT(&htim2, TIM_CHANNEL_2);

- IC = Input Capture

- IT = Interrupt mode

When an edge is captured, STM32 automatically calls:

HAL_TIM_IC_CaptureCallback()

This is where you'll process the capture values.

Read the captured timer count:

HAL_TIM_ReadCapturedValue(&htim2, TIM_CHANNEL_2);

This returns the value stored in CCR2.

# Firmware Flow

Start Timer

↓

Enable Input Capture Interrupt

↓

Send Trigger Pulse

↓

Rising Edge Interrupt

↓

Save Start Time

↓

Switch to Falling Edge

↓

Falling Edge Interrupt

↓

Save End Time

↓

Calculate Pulse Width

↓

Calculate Distance

↓

Display Distance