## UART Data Logging

Your STM32 will send data like:

Distance: 18.3 cm

Water Level: 31.7 cm

Level: 64%

Status: NORMAL

-----------------------

to a PC every second.

This demonstrates:

- UART communication

- printf() redirection

- Serial monitoring

- Industrial-style data logging

These are skills that interviewers commonly ask about and are highly valuable on an embedded firmware resume.

Today 1:16 PM

next

Excellent. Now we'll implement UART Logging, one of the most useful features for interviews.

This feature demonstrates:

- UART peripheral configuration

- Data formatting

- Serial communication

- Embedded debugging

# Lesson 12: UART Data Logging

## Why do we need UART?

Suppose the LCD shows:

Level : 64%

Dist  : 18.3cm

That's useful locally.

But what if an operator sitting at a computer wants to monitor the tank?

We use UART.

STM32  -------- UART --------> PC

The PC can receive live data using software like:

- Docklight (you already have it)

- Tera Term

- PuTTY

- RealTerm

# Step 1: Configure USART1 in CubeMX

Configure:

| Parameter | Value |
| --- | --- |
| Mode | Asynchronous |
| Baud Rate | 115200 |
| Word Length | 8 Bits |
| Parity | None |
| Stop Bits | 1 |
| Hardware Flow Control | None |

Generate the code.

CubeMX will create:

UART_HandleTypeDef huart1;

# Step 2: Include Header

In main.c:

#include <stdio.h>

#include <string.h>

stdio.h → sprintf()

string.h → strlen()

# Step 3: Create UART Buffer

Near your global variables:

char uartBuffer[100];

Why 100?

Because one complete message is much shorter than 100 characters, so this is a safe buffer size.

# Step 4: Create the Message

Suppose:

Distance = 18.3

Water Level = 31.7

Level = 64%

Create the string:

sprintf(uartBuffer,

"Distance: %.1f cm\r\n"

"Water Level: %.1f cm\r\n"

"Level: %.0f %%\r\n\r\n",

distance,

waterLevel,

levelPercent);

Let's understand it.

## Why \r\n?

PC serial terminals expect:

Carriage Return (\r)

+

Line Feed (\n)

Without it:

Distance:18.3Water:31.7Level:64

Everything appears on one line.

With \r\n:

Distance:18.3

Water:31.7

Level:64

## Why %.1f?

18.3456

becomes

18.3

Cleaner for the user.

## Why %.0f %%?

64.3

becomes

64%

# Step 5: Send Using UART

HAL_UART_Transmit(&huart1,

(uint8_t*)uartBuffer,

strlen(uartBuffer),

HAL_MAX_DELAY);

Let's understand each parameter.

### Parameter 1

&huart1

Which UART?

USART1.

### Parameter 2

(uint8_t*)uartBuffer

UART transmits bytes.

A string is a sequence of characters.

We cast the character pointer to a byte pointer because the HAL function expects a uint8_t *.

### Parameter 3

strlen(uartBuffer)

How many bytes should be transmitted?

If the string contains 52 characters:

52 bytes transmitted

### Parameter 4

HAL_MAX_DELAY

Wait until transmission completes.

# Data Flow

Distance

↓

sprintf()

↓

ASCII String

↓

HAL_UART_Transmit()

↓

USART1

↓

USB-UART

↓

PC

# Expected Output in Docklight

Distance: 18.3 cm

Water Level: 31.7 cm

Level: 64%

Distance: 18.2 cm

Water Level: 31.8 cm

Level: 64%

Distance: 17.9 cm

Water Level: 32.1 cm

Level: 64%

# Where Should This Code Go?

Inside your while(1) loop, after calculating the values:

while (1)

{

HCSR04_Trigger();

HAL_Delay(100);

float distance = HCSR04_GetDistance();

float waterLevel = TANK_HEIGHT_CM - distance;

float levelPercent =

(waterLevel * 100.0f) / TANK_HEIGHT_CM;

// LCD Update

// Alarm Logic

sprintf(uartBuffer,

"Distance: %.1f cm\r\n"

"Water Level: %.1f cm\r\n"

"Level: %.0f %%\r\n\r\n",

distance,

waterLevel,

levelPercent);

HAL_UART_Transmit(&huart1,

(uint8_t *)uartBuffer,

strlen(uartBuffer),

HAL_MAX_DELAY);

}

# Interview Questions

### Q1. Why use sprintf() before transmitting?

Answer:

UART transmits bytes, not floating-point variables. sprintf() converts numerical values into formatted ASCII strings that can be sent over UART.

### Q2. Why cast uartBuffer to uint8_t *?

Answer:

HAL_UART_Transmit() expects a pointer to bytes (uint8_t *). A C string is stored as an array of char, so we cast it to the required type.

### Q3. Why use strlen()?

Answer:

The HAL needs to know how many bytes to send. strlen() returns the number of characters in the formatted string.