# Lesson 11: Alarm Logic Design

## Requirement

The system should:

- 🟢 Green LED → Water level is normal.

- 🔴 Red LED → Water level is low.

- 🔊 Buzzer → Water level is critically low.

- Later, the threshold values will be stored in EEPROM.

# Step 1: Define Thresholds

At the top of main.c, inside the USER CODE section:

#define LOW_LEVEL_THRESHOLD       20.0f    // 20%

#define CRITICAL_LEVEL_THRESHOLD  10.0f    // 10%

### Why percentages?

Suppose one customer has:

- 50 cm tank

Another has:

- 200 cm tank

If we use centimeters:

Low Level = 10 cm

It works for one tank but not the other.

Using percentages:

20%

works for every tank.

# Step 2: Alarm Conditions

We already calculate:

levelPercent

Now compare it.

### Case 1: Normal

Level = 75%

75 > 20

Normal

Green LED ON

Red LED OFF

Buzzer OFF

### Case 2: Low

Level = 15%

15 < 20

Low Level

Green OFF

Red ON

Buzzer OFF

### Case 3: Critical

Level = 5%

5 < 10

Green OFF

Red ON

Buzzer ON

# Step 3: Code

Inside your while(1) after calculating levelPercent:

if(levelPercent <= CRITICAL_LEVEL_THRESHOLD)

{

HAL_GPIO_WritePin(RED_LED_GPIO_Port,

RED_LED_Pin,

GPIO_PIN_SET);

HAL_GPIO_WritePin(GREEN_LED_GPIO_Port,

GREEN_LED_Pin,

GPIO_PIN_RESET);

HAL_GPIO_WritePin(BUZZER_GPIO_Port,

BUZZER_Pin,

GPIO_PIN_SET);

}

else if(levelPercent <= LOW_LEVEL_THRESHOLD)

{

HAL_GPIO_WritePin(RED_LED_GPIO_Port,

RED_LED_Pin,

GPIO_PIN_SET);

HAL_GPIO_WritePin(GREEN_LED_GPIO_Port,

GREEN_LED_Pin,

GPIO_PIN_RESET);

HAL_GPIO_WritePin(BUZZER_GPIO_Port,

BUZZER_Pin,

GPIO_PIN_RESET);

}

else

{

HAL_GPIO_WritePin(RED_LED_GPIO_Port,

RED_LED_Pin,

GPIO_PIN_RESET);

HAL_GPIO_WritePin(GREEN_LED_GPIO_Port,

GREEN_LED_Pin,

GPIO_PIN_SET);

HAL_GPIO_WritePin(BUZZER_GPIO_Port,

BUZZER_Pin,

GPIO_PIN_RESET);

}

# Why do we check the critical condition first?

Suppose:

Level = 5%

If we checked:

if(levelPercent <= 20)

first, then:

5 <= 20

TRUE

The program would never reach the critical alarm.

So always check the most restrictive condition first:

Critical

↓

Low

↓

Normal

# Flowchart

Read Level

│

▼

Is Level ≤ 10% ?

│          │

Yes          No

│            │

▼            ▼

Red ON           Is Level ≤ 20% ?

Green OFF          │          │

Buzzer ON        Yes          No

│            │

▼            ▼

Red ON       Green ON

Green OFF      Red OFF

Buzzer OFF     Buzzer OFF

# Test Cases

| Water Level | Green LED | Red LED | Buzzer |
| --- | --- | --- | --- |
| 90% | ON | OFF | OFF |
| 50% | ON | OFF | OFF |
| 18% | OFF | ON | OFF |
| 9% | OFF | ON | ON |

When testing your project, verify each of these cases.

# Interview Questions

### Q1. Why did you use percentages instead of centimeters?

Answer:

Using percentages makes the alarm logic independent of the physical tank size. The same firmware can work with tanks of different heights by changing only the tank height parameter.

### Q2. Why check the critical level first?

Answer:

Because the critical range is a subset of the low-level range. If the low-level condition were checked first, the critical alarm would never be executed.