# Enable Input Capture Interrupt

In main.c, after starting TIM6, also start TIM2 Input Capture.

HAL_TIM_Base_Start(&htim6);

HAL_TIM_IC_Start_IT(&htim2, TIM_CHANNEL_2);

### What does this do?

HAL_TIM_IC_Start_IT() means:

- Enable Input Capture

- Enable Interrupt

- Use TIM2 Channel 2

Now every capture event will automatically call:

HAL_TIM_IC_CaptureCallback()

# Step 3: Declare Variables

In ultrasonic.c:

uint32_t IC_Value1 = 0;

uint32_t IC_Value2 = 0;

uint32_t Difference = 0;

uint8_t Is_First_Captured = 0;

float Distance = 0;

Let's understand each one.

## IC_Value1

Stores the timer value at the rising edge.

Example:

Echo

_____|‾‾‾‾‾‾‾

↑

IC_Value1 = 1500

## IC_Value2

Stores the timer value at the falling edge.

Echo

_____|‾‾‾‾|____

↑

IC_Value2 = 3482

## Difference

Difference

=

3482

-

1500

=

1982

This is the Echo pulse width.

## Is_First_Captured

This variable tells us:

0

Waiting for Rising Edge

or

1

Waiting for Falling Edge

It works like a simple state machine.

# Step 4: The Callback Function

# void HAL_TIM_IC_CaptureCallback(TIM_HandleTypeDef *htim)

# {

# if (htim->Instance == TIM2)

# {

# if (Is_First_Captured == 0)

# {

# IC_Value1 = HAL_TIM_ReadCapturedValue(htim, TIM_CHANNEL_2);

# Is_First_Captured = 1;

# __HAL_TIM_SET_CAPTUREPOLARITY(htim,

# TIM_CHANNEL_2,

# TIM_INPUTCHANNELPOLARITY_FALLING);

# }

# else

# {

# IC_Value2 = HAL_TIM_ReadCapturedValue(htim, TIM_CHANNEL_2);

# if (IC_Value2 > IC_Value1)

# {

# Difference = IC_Value2 - IC_Value1;

# }

# else

# {

# Difference = (65535 - IC_Value1) + IC_Value2 + 1;

# }

# Distance = Difference / 58.0f;

# Is_First_Captured = 0;

# __HAL_TIM_SET_CAPTUREPOLARITY(htim,

# TIM_CHANNEL_2,

# TIM_INPUTCHANNELPOLARITY_RISING);

# }

# }

# }

Add this to ultrasonic.c:

void HAL_TIM_IC_CaptureCallback(TIM_HandleTypeDef *htim)

{

}

### Why is it called automatically?

When TIM2 captures an edge, the interrupt occurs.

The HAL interrupt handler eventually calls:

HAL_TIM_IC_CaptureCallback()

You don't call it yourself.

# Step 5: Check Which Timer Generated the Interrupt

if(htim->Instance == TIM2)

{

}

Why?

Because multiple timers can generate Input Capture interrupts.

Example:

TIM2

TIM3

TIM15

We only want TIM2.

# Step 6: Check First Capture

if(Is_First_Captured == 0)

{

}

Meaning:

Still waiting for Rising Edge

# Step 7: Read Rising Edge

IC_Value1 = HAL_TIM_ReadCapturedValue(htim, TIM_CHANNEL_2);

Suppose timer count:

1520

Now:

IC_Value1 = 1520

# Step 8: Tell Timer to Capture Falling Edge Next

__HAL_TIM_SET_CAPTUREPOLARITY(

htim,

TIM_CHANNEL_2,

TIM_INPUTCHANNELPOLARITY_FALLING);

Now the timer ignores rising edges and waits for the falling edge.

# Step 9

Is_First_Captured = 1;

Meaning:

Rising Edge Done

Now waiting for Falling Edge

# Flow So Far

Trigger Pulse

↓

Echo Rising Edge

↓

Capture Timer

↓

Store IC_Value1

↓

Change to Falling Edge

↓

Wait

# What happens after the falling edge?

Suppose the Echo signal is:

Echo

_______|‾‾‾‾‾‾‾‾|_______

↑        ↑

Start     End

At the falling edge:

- TIM2 stores the current counter value in CCR2

- Interrupt occurs

- HAL_TIM_IC_CaptureCallback() executes again

This time:

Is_First_Captured == 1

So we enter:

else

{

}

# Step 1: Read the second capture value

IC_Value2 = HAL_TIM_ReadCapturedValue(htim, TIM_CHANNEL_2);

Suppose:

Timer Count

3482

Now:

IC_Value2 = 3482

# Step 2: Calculate pulse width

Most examples show:

Difference = IC_Value2 - IC_Value1;

Example:

IC_Value1 = 1500

IC_Value2 = 3482

Then:

Difference

=

3482

-

1500

=

1982 µs

Looks good...

But what if the timer overflows?

# Understanding Timer Overflow

Remember:

TIM2 counts like this:

0

1

2

...

65534

65535

↓

0

1

2

Suppose:

Rising edge:

65000

Falling edge:

500

Now if we do:

Difference = 500 - 65000;

Result?

Negative number ❌

But the pulse width isn't negative.

# Correct Way

We must check:

if (IC_Value2 > IC_Value1)

{

Difference = IC_Value2 - IC_Value1;

}

else

{

Difference = (65535 - IC_Value1) + IC_Value2 + 1;

}

## Let's verify the formula

Example:

Start = 65000

End = 500

Remaining counts before overflow:

65535 - 65000

=

535

Then:

535

+

500

+

1

=

1036 µs

Correct.

# Why add +1?

Because the timer counts both:

65535

↓

0

There is one timer tick between those values.

This is a common way to compute elapsed counts across a wraparound.

# Step 3: Convert pulse width to distance

We already know:

Distance(cm)

=

Time / 58

So:

Distance = Difference / 58.0f;

Suppose:

Difference

=

1982

Distance:

1982 / 58

=

34.17 cm

# Step 4: Reset everything

After one complete measurement:

Is_First_Captured = 0;

Meaning:

Ready for next Rising Edge

# Step 5: Change polarity back

Now configure TIM2 to capture the next rising edge.

__HAL_TIM_SET_CAPTUREPOLARITY(

htim,

TIM_CHANNEL_2,

TIM_INPUTCHANNELPOLARITY_RISING);

Otherwise:

TIM2 will continue waiting for falling edges forever.

# Complete Flow

Trigger

↓

Rising Edge

↓

IC_Value1

↓

Change to Falling

↓

Falling Edge

↓

IC_Value2

↓

Difference

↓

Distance

↓

Reset Flag

↓

Change to Rising

↓

Next Measurement