## Save LowLevelThreshold into EEPROM

Now we'll connect your menu setting to the EEPROM.

The desired behavior is:

Normal Mode

↓

Press SET

↓

Settings Mode

↓

UP / DOWN

↓

Low Level = 25%

↓

Press SET

↓

Save 25% to EEPROM

↓

Return to Normal Mode

### 1. First, use an EEPROM address

Let's reserve:

#define EEPROM_LOW_LEVEL_ADDR  0x0000

Put this in main.c inside:

/* USER CODE BEGIN PD */

So:

/* USER CODE BEGIN PD */

#define EEPROM_LOW_LEVEL_ADDR  0x0000

/* USER CODE END PD */

### 2. When SET is pressed, save the value

Your current code has:

if(HAL_GPIO_ReadPin(SET_BUTTON_GPIO_Port,

SET_BUTTON_Pin)==GPIO_PIN_RESET)

{

HAL_Delay(100);

settingMode = !settingMode;

}

Change it to:

if(HAL_GPIO_ReadPin(SET_BUTTON_GPIO_Port,

SET_BUTTON_Pin) == GPIO_PIN_RESET)

{

HAL_Delay(100);

if(settingMode == 1)

{

EEPROM_WriteByte(EEPROM_LOW_LEVEL_ADDR,

LowLevelThreshold);

}

settingMode = !settingMode;

}

### Understand what happens

Suppose:

LowLevelThreshold = 25;

When you press SET while you're already in settings mode:

EEPROM_WriteByte(0x0000, 25);

So the EEPROM stores:

Address       Data

0x0000   →    25

Then:

settingMode = !settingMode;

changes:

1 → 0

and you return to normal mode.

# Step 18 — Read the value when the STM32 starts

Now we need to restore the saved value after power loss.

You previously had this:

/*

LowLevelThreshold = EEPROM_ReadByte(0x0000);

if(LowLevelThreshold == 0xFF)

{

LowLevelThreshold = 20;

}

*/

Now uncomment it and use our address constant:

LowLevelThreshold = EEPROM_ReadByte(EEPROM_LOW_LEVEL_ADDR);

if(LowLevelThreshold == 0xFF)

{

LowLevelThreshold = 20;

}

Put this in:

/* USER CODE BEGIN 2 */

after:

MX_I2C1_Init();

because the EEPROM communicates through I²C.

For example:

MX_GPIO_Init();

MX_I2C1_Init();

MX_TIM2_Init();

MX_USART1_UART_Init();

MX_TIM6_Init();

/* USER CODE BEGIN 2 */

lcd_init();

lcd_clear();

LowLevelThreshold = EEPROM_ReadByte(EEPROM_LOW_LEVEL_ADDR);

if(LowLevelThreshold == 0xFF)

{

LowLevelThreshold = 20;

}

HAL_TIM_Base_Start(&htim6);

HAL_TIM_IC_Start_IT(&htim2, TIM_CHANNEL_2);

# Important: Why check 0xFF?

A freshly erased/unwritten EEPROM location commonly reads:

0xFF

So:

if(LowLevelThreshold == 0xFF)

means:

"There is no valid threshold stored yet."

Therefore we choose a default:

LowLevelThreshold = 20;

So the first time your system starts:

EEPROM = 0xFF

↓

No saved setting

↓

LowLevelThreshold = 20%

Later, suppose you set:

25%

and press SET:

EEPROM = 25

Power OFF → Power ON:

EEPROM → 25

↓

LowLevelThreshold = 25%

## One important thing before you test

Your EEPROM is an I²C device, so make sure the EEPROM wiring and EEPROM_ADDR in your eeprom.c match the actual AT24C32 hardware address.

If your EEPROM driver currently has something like:

#define EEPROM_ADDR 0xA0

don't change it yet.

### Your next practical test

Don't add the Critical threshold yet.

Test only this:

- Power ON → threshold should be 20% initially.

- Enter settings.

- Change it to 25%.

- Press SET to save.

- Power OFF.

- Power ON.

- Check whether LCD/settings show 25%.

If that works, we've proved the complete:

Button → RAM → EEPROM → Power loss → EEPROM → RAM

chain.

Then we'll add Critical Level threshold using a second EEPROM address.

Yes. Before testing Step 17, you should physically connect the AT24C32 EEPROM to your STM32. Otherwise the EEPROM read/write functions cannot work.

Since your project uses I2C1, we'll connect the AT24C32 to your STM32's I2C1 pins.

### AT24C32 → STM32

From your current configuration, you have:

- PA9 = I2C1_SCL

- PA10 = I2C1_SDA (assuming this is what you configured for I2C1)

The connections are:

| AT24C32 | STM32 |
| --- | --- |
| VCC | 3.3 V |
| GND | GND |
| SDA | I2C1 SDA |
| SCL | I2C1 SCL |
| WP | GND |
| A0 | GND |
| A1 | GND |
| A2 | GND |

Important: I want to verify your exact STM32 pin mapping before you wire it, because you've already configured PA9 for I2C1 SCL and we should confirm the SDA pin from your CubeMX configuration rather than assume it.

Also, I2C needs pull-up resistors on SDA and SCL. Many AT24C32 modules already have them; a bare IC may not.

### Don't change the code yet.

First, connect the EEPROM and verify communication. We can make a small I2C test that writes one byte and reads it back:

STM32

│

│ I2C

▼

AT24C32

│

├── Write 25

│

└── Read 25

↓

LCD/UART

If we can successfully read back 25, then we'll proceed with the threshold-saving functionality.