Now we'll implement the EEPROM Read function. This is the opposite of writing and introduces an important I²C concept: repeated start.

# Lesson 14: Reading One Byte from AT24C32 EEPROM

## Goal

We want to read a value stored in EEPROM.

For example:

Address 0x0000 = 30

When we execute:

value = EEPROM_ReadByte(0x0000);

we should get:

value = 30

# How does EEPROM know what to read?

Imagine asking a librarian:

"Give me the book."

The librarian asks:

"Which shelf?"

Similarly, the EEPROM needs to know:

- Which memory address?

- Then it can return the data stored there.

So reading is actually a two-step process.

# Step 1: Send the Memory Address

Suppose we want to read:

Address = 0x0123

We first send only the address:

START

↓

Device Address (Write)

↓

Address MSB

↓

Address LSB

Notice:

No data is sent.

We're just telling the EEPROM:

"The location I want is 0x0123."

# Step 2: Read the Data

Now the STM32 says:

"Okay, now send me the data."

The communication becomes:

Repeated START

↓

Device Address (Read)

↓

EEPROM sends Data

↓

STOP

# Complete Transaction

The datasheet shows it like this:

STM32                          EEPROM

--------------------------------------------

START

-------->

0xA0 (Write Address)

-------->

ACK

<--------

Address MSB

-------->

ACK

<--------

Address LSB

-------->

ACK

<--------

REPEATED START

-------->

0xA1 (Read Address)

-------->

ACK

<--------

Data

<--------

NACK

STOP

This is why reading is slightly more complex than writing.

# STM32 HAL Makes It Easy

Instead of manually generating a repeated START, HAL provides:

HAL_I2C_Mem_Read()

This function does all the steps internally.

# EEPROM_ReadByte()

In eeprom.c, add:

uint8_t EEPROM_ReadByte(uint16_t address)

{

uint8_t data;

HAL_I2C_Mem_Read(

&hi2c1,

EEPROM_ADDR,

address,

I2C_MEMADD_SIZE_16BIT,

&data,

1,

HAL_MAX_DELAY);

return data;

}

# Understanding Every Parameter

Let's examine the function:

HAL_I2C_Mem_Read(

&hi2c1,

EEPROM_ADDR,

address,

I2C_MEMADD_SIZE_16BIT,

&data,

1,

HAL_MAX_DELAY);

## Parameter 1

&hi2c1

Use the I2C1 peripheral.

Exactly like HAL_I2C_Master_Transmit().

## Parameter 2

EEPROM_ADDR

EEPROM device address.

For AT24C32:

#define EEPROM_ADDR (0x50 << 1)

## Parameter 3

address

The EEPROM memory location.

Example:

0x0000

or

0x0123

## Parameter 4

I2C_MEMADD_SIZE_16BIT

This tells HAL:

"The EEPROM uses a 16-bit memory address."

AT24C32 has:

4096 Bytes

An 8-bit address can represent only:

0–255

which is not enough.

A 16-bit address can represent:

0–65535

which easily covers all EEPROM locations.

## Parameter 5

&data

The EEPROM will place the received byte into this variable.

Initially:

data = ?

After reading:

data = 30

## Parameter 6

1

Read one byte.

If we wanted to read 10 bytes:

HAL_I2C_Mem_Read(

...

buffer,

10,

...

);

## Parameter 7

HAL_MAX_DELAY

Maximum timeout.

# Example

Suppose EEPROM contains:

| Address | Value |
| --- | --- |
| 0x0000 | 30 |
| 0x0001 | 10 |

Now:

uint8_t low;

low = EEPROM_ReadByte(0x0000);

Result:

low = 30

Similarly:

uint8_t critical;

critical = EEPROM_ReadByte(0x0001);

Result:

critical = 10

# Why did we use HAL_I2C_Mem_Read() instead of HAL_I2C_Master_Receive()?

Because HAL_I2C_Mem_Read() is specifically designed for memory devices like EEPROMs. It automatically:

- Sends the device address in write mode.

- Sends the memory address (8-bit or 16-bit).

- Generates the repeated START condition.

- Switches to read mode.

- Receives the requested data.

If you used HAL_I2C_Master_Receive() directly, you would first have to send the memory address yourself before reading.

## Interview Question

Interviewer: Why do you use I2C_MEMADD_SIZE_16BIT with the AT24C32?

Answer:

The AT24C32 uses a 16-bit internal memory address because it contains 4096 bytes of memory. An 8-bit address can access only 256 locations, so the EEPROM requires two address bytes (MSB and LSB). The I2C_MEMADD_SIZE_16BIT parameter tells the HAL to transmit both address bytes before reading the data.