# Lesson 13: EEPROM (AT24C32) - Save Alarm Settings

## Why do we need EEPROM?

Imagine your tank alarm threshold is set to:

Low Level      = 20%

Critical Level = 10%

Now the power goes OFF.

When power comes back:

- Without EEPROM ❌

Low Level = 0%

Critical = 0%

All settings are lost because RAM is volatile.

- With EEPROM ✅

Low Level = 20%

Critical = 10%

The settings are restored automatically.

Excellent catch. This is a very common confusion between program memory (Flash/ROM) and RAM. Let's clear it up.

## First, where is your code stored?

When you compile and program your STM32:

main.c

ultrasonic.c

eeprom.c

all of this is stored in the STM32 Flash memory (often called program memory or ROM).

Flash Memory

-------------------------

Program Code

Functions

Constants

-------------------------

When power is OFF:

✅ Your code does not disappear.

## Then what disappears?

Suppose in your code you have:

uint8_t LowLevelThreshold = 20;

This variable is copied into RAM when the MCU starts executing.

Flash

--------------------

Initial value = 20

--------------------

↓ Reset

RAM

--------------------

LowLevelThreshold = 20

--------------------

## Now suppose the user changes it

Imagine you press the UP button:

LowLevelThreshold = 35;

Now RAM contains:

RAM

LowLevelThreshold = 35

But Flash still contains:

Flash

Initial value = 20

## Now power goes OFF

RAM loses everything.

RAM

XXXXXXXXXXXX

Data Lost

When power comes back:

The MCU starts from Flash again.

So:

uint8_t LowLevelThreshold = 20;

is copied into RAM.

Result:

LowLevelThreshold = 20

Your change to 35 is lost.

# Why not save it in Flash?

Technically, STM32 Flash can be written during program execution.

However:

- Flash has a limited number of write/erase cycles (typically around 10,000 per page on many STM32 devices).

- Erasing Flash is slow and is done page-by-page.

- It is intended mainly for storing the program.

EEPROM is designed specifically for storing small amounts of frequently changing configuration data.

## Example

Suppose your firmware contains:

uint8_t LowLevelThreshold = 20;

After programming the MCU:

Flash = 20

User changes it:

RAM = 35

Power OFF:

RAM → Lost

Power ON:

Flash → RAM

20 is copied again

So the user setting 35 disappears.

## With EEPROM

Instead of only changing RAM:

LowLevelThreshold = 35;

we also write:

EEPROM_WriteByte(0x0000, 35);

Now:

EEPROM

Address 0x0000

35

Power OFF:

EEPROM still contains:

35

Power ON:

During initialization:

LowLevelThreshold = EEPROM_ReadByte(0x0000);

Now RAM becomes:

35

The user's setting is restored.

## Interview Question

Interviewer: If Flash memory is non-volatile, why use an external EEPROM?

Good Answer:

The firmware stored in Flash does not disappear after power loss. However, configuration values that change during runtime, such as user-set alarm thresholds, are stored in RAM while the program is running. RAM is volatile, so those changes are lost when power is removed. An EEPROM provides non-volatile storage for such user-configurable data, allowing it to be restored after a restart.

This distinction between program code in Flash and runtime data in RAM is fundamental in embedded systems and is asked frequently in interviews.

No problem. Let's forget EEPROM for a minute and think about your actual project.

## Scenario 1: No EEPROM

Suppose your firmware has:

uint8_t LowLevelThreshold = 20;

This means:

"Turn ON the red LED when the water level is below 20%."

You program the STM32.

Everything works.

Now imagine you add three buttons:

- SET

- UP

- DOWN

The factory operator wants the alarm at 30% instead of 20%.

He presses:

SET

UP

UP

UP

...

Now your program changes:

LowLevelThreshold = 30;

The alarm now works at 30%.

So far, so good.

### Then power goes OFF.

Power comes back ON.

What happens?

Your program starts from the beginning again.

The line:

uint8_t LowLevelThreshold = 20;

runs again.

Now:

Threshold = 20%

The operator has to press the buttons again to make it 30%.

Every single time power is lost.

This is inconvenient.

# Scenario 2: With EEPROM

The operator changes:

20%

to

30%

Your code does:

LowLevelThreshold = 30;

EEPROM_WriteByte(0x0000, 30);

Now the EEPROM permanently stores:

Address 0x0000 = 30

Power goes OFF.

Power comes ON.

During startup:

LowLevelThreshold = EEPROM_ReadByte(0x0000);

Now:

Threshold = 30%

The operator doesn't need to configure it again.

# The important point

If your threshold never changes, like this:

#define LOW_LEVEL_THRESHOLD 20

then you do NOT need EEPROM.

EEPROM is unnecessary because the value is fixed in your firmware.

EEPROM becomes useful only when the user can change settings while the device is running.

Examples:

- Alarm threshold ✔

- Tank height ✔

- Device ID ✔

- Calibration values ✔

- Wi-Fi password ✔

- User preferences ✔

# In your current project

Right now, have we written any code like this?

if(SET button pressed)

{

LowLevelThreshold++;

}

No.

So at this moment, EEPROM has no purpose.

That's why your confusion is completely understandable.

## My recommendation

For an interview-worthy project, we should make those three buttons actually useful:

- SET → Enter settings mode.

- UP → Increase the low-level threshold.

- DOWN → Decrease the threshold.

- Press SET again → Save the new threshold to EEPROM.

Then EEPROM becomes a meaningful part of the project rather than just being added for the sake of using it.

Without adjustable settings, I would not include EEPROM in the project because it doesn't solve a real problem.

This is a much stronger design, and an interviewer is likely to appreciate that you used EEPROM because there was a genuine requirement for persistent user configuration, not just because the component was available.

"My code is already stored in ROM. Why do I need EEPROM?"

The answer is:

- ✅ Program code is stored in Flash (ROM) and never disappears.

- ✅ User settings changed while the program is running exist only in RAM unless you save them somewhere permanent.

- ✅ EEPROM is that permanent storage.

A simple analogy:

Imagine your firmware is a restaurant menu.

The menu (Flash) always says:

Low Level Threshold = 20%

One customer says:

"Change it to 30%."

The waiter writes 30% on a whiteboard (RAM).

As long as the restaurant has power, everyone sees 30%.

But when the restaurant closes (power OFF), the whiteboard is erased.

Next morning, the waiter again reads the printed menu:

20%

If instead the waiter writes 30% in a register book (EEPROM), then even after closing and reopening, he reads the register book and still knows it's 30%.

## This is exactly how your project will work

First power ON:

Default Low Level = 20%

User enters settings:

SET

UP

UP

UP

New Threshold = 30%

Your code:

LowLevelThreshold = 30;

EEPROM_WriteByte(0x0000, 30);

Power OFF.

Power ON again:

LowLevelThreshold = EEPROM_ReadByte(0x0000);

Result:

Low Level = 30%

No need for the user to configure it again.

# Memory Types

RAM

│

├── Very Fast

├── Data Lost after Power OFF

└── Variables

EEPROM

│

├── Slower

├── Data Retained

└── Configuration Data

# Our Project Architecture

Buttons

│

▼

User changes threshold

│

▼

Write to EEPROM

│

--------------------

Power OFF

--------------------

│

▼

Read from EEPROM

│

▼

Alarm works with old settings

# What will we store?

Instead of hardcoding:

#define LOW_LEVEL_THRESHOLD 20

We'll use variables:

uint8_t LowLevelThreshold;

uint8_t CriticalLevelThreshold;

Initially they contain:

LowLevelThreshold      = 20

CriticalLevelThreshold = 10

These values will be saved in EEPROM.

# EEPROM Memory Map

AT24C32 has many memory locations.

We'll reserve the first few bytes.

| EEPROM Address | Data |
| --- | --- |
| 0x0000 | Low Level Threshold |
| 0x0001 | Critical Threshold |

Simple and easy to understand.

# Step 1: Create EEPROM Driver

Create new files:

Core

├── Inc

│      eeprom.h

│

└── Src

eeprom.c

Exactly like you did for ultrasonic.c.

# Step 2: Create eeprom.h

Write this yourself:

#ifndef EEPROM_H

#define EEPROM_H

#include "main.h"

void EEPROM_WriteByte(uint16_t address, uint8_t data);

uint8_t EEPROM_ReadByte(uint16_t address);

#endif

Notice something:

This is exactly the same style as ultrasonic.h.

Every driver has:

- Header (.h)

- Source (.c)

# Step 3: EEPROM Device Address

AT24C32 uses I²C.

Its 7-bit address is:

1010000

HAL expects the address shifted left by one bit:

#define EEPROM_ADDR 0xA0

Interview Tip: Many EEPROM datasheets list the 7-bit address (0x50), while STM32 HAL functions use the 8-bit address (0xA0) because the R/W bit occupies the least significant bit during transmission. This is a very common interview question.

# Step 4: Write a Byte

In eeprom.c:

#include "eeprom.h"

extern I2C_HandleTypeDef hi2c1;

void EEPROM_WriteByte(uint16_t address, uint8_t data)

{

uint8_t buffer[3];

buffer[0] = (address >> 8);      // Address MSB

buffer[1] = address & 0xFF;      // Address LSB

buffer[2] = data;                // Data

HAL_I2C_Master_Transmit(&hi2c1,

EEPROM_ADDR,

buffer,

3,

HAL_MAX_DELAY);

HAL_Delay(5);

}

extern I2C_HandleTypeDef hi2c1; whats this

Excellent. This is one of the most important C concepts in STM32 programming. Once you understand extern, you'll understand why STM32 projects are split into multiple .c files.

Let's go step by step.

# First, where is hi2c1 created?

After CubeMX generates code, open i2c.c.

You'll find something like:

I2C_HandleTypeDef hi2c1;

This is the actual variable.

Think of it as:

i2c.c

I2C_HandleTypeDef hi2c1;

↑

Memory is allocated here

There is only one hi2c1 in your entire program.

# What is I2C_HandleTypeDef?

It is a structure (struct) defined by the HAL library.

Simplified:

typedef struct

{

I2C_TypeDef *Instance;

I2C_InitTypeDef Init;

HAL_LockTypeDef Lock;

__IO HAL_I2C_StateTypeDef State;

...

} I2C_HandleTypeDef;

This structure stores everything about I2C1:

- Which peripheral? (I2C1)

- Clock settings

- Speed

- Addressing mode

- Current state

- Error flags

- etc.

# Why do we need extern?

Suppose your project has:

i2c.c

main.c

eeprom.c

hi2c1 is created only in:

// i2c.c

I2C_HandleTypeDef hi2c1;

Now in eeprom.c you write:

HAL_I2C_Master_Transmit(&hi2c1, ...);

The compiler asks:

"Where is hi2c1? I don't know this variable."

So you tell it:

extern I2C_HandleTypeDef hi2c1;

Meaning:

"Don't create a new variable. Trust me—this variable already exists in another source file."

# Think of it like a TV

Imagine:

Your TV is in the living room.

Living Room

TV

Your bedroom doesn't have a TV.

Instead of buying another TV, you say:

"Use the TV in the living room."

That's what extern does.

It doesn't create a second TV.

It simply tells the compiler where to find the existing one.

# Without extern

Suppose you write in eeprom.c:

I2C_HandleTypeDef hi2c1;

Now you have:

i2c.c

hi2c1

and

eeprom.c

hi2c1

Two different variables!

Which one did CubeMX initialize?

The one in i2c.c.

The one in eeprom.c is uninitialized.

Your EEPROM driver would use the wrong handle, and I²C communication would fail.

# With extern

extern I2C_HandleTypeDef hi2c1;

Now:

main.c

\

eeprom.c -----> hi2c1 <----- i2c.c

/

other.c

Every file uses the same hi2c1.

# Why do we write &hi2c1?

The function prototype is:

HAL_I2C_Master_Transmit(

I2C_HandleTypeDef *hi2c,

...

);

Notice the *.

It expects a pointer to the handle.

So we pass its address:

&hi2c1

This lets the HAL function access and update the same handle structure.

# Interview Question

Interviewer: What does extern I2C_HandleTypeDef hi2c1; mean?

Good Answer:

hi2c1 is defined in i2c.c by CubeMX. When another source file, such as eeprom.c, needs to use the same I²C handle, we declare it with extern. This tells the compiler that the variable is defined elsewhere and avoids creating a second copy. As a result, all modules share the same initialized I²C peripheral handle.

## General Rule

You'll see this pattern throughout STM32 projects:

extern I2C_HandleTypeDef hi2c1;

extern UART_HandleTypeDef huart1;

extern TIM_HandleTypeDef htim2;

extern TIM_HandleTypeDef htim6;

extern ADC_HandleTypeDef hadc;

All of these are defined once (usually by CubeMX) and referenced with extern in other source files whenever needed.

## Figure 1: Device Address

At the top of your screenshot, you see:

1 0 1 0 A2 A1 A0 R/W

This is the 8-bit address sent on the I²C bus.

Let's break it down.

Bit7 Bit6 Bit5 Bit4 Bit3 Bit2 Bit1 Bit0

1    0    1    0   A2   A1   A0   R/W

### What are A2, A1, A0?

These are hardware address pins on the EEPROM.

On most AT24C32 modules, they are connected to GND.

So,

A2 = 0

A1 = 0

A0 = 0

Now the address becomes:

1 0 1 0 0 0 0 R/W

## Case 1: Write Operation

During write:

R/W = 0

Address becomes:

10100000

Binary to Hex:

1010 0000

=

0xA0

This is why we write:

#define EEPROM_ADDR 0xA0

## Case 2: Read Operation

During read:

R/W = 1

Address becomes:

10100001

=

0xA1

## Then why don't we write 0xA1 for read?

Because the STM32 HAL functions handle the R/W bit automatically.

When you call:

HAL_I2C_Master_Transmit(&hi2c1, 0xA0, ...);

HAL sends:

10100000

When you call:

HAL_I2C_Master_Receive(&hi2c1, 0xA0, ...);

HAL automatically changes the last bit to 1 and sends:

10100001

So you always pass:

0xA0

to both transmit and receive functions.

# Figure 2: Byte Write

Now let's understand the timing diagram.

You see:

START

↓

DEVICE ADDRESS

↓

FIRST WORD ADDRESS

↓

SECOND WORD ADDRESS

↓

DATA

↓

STOP

Let's say you execute:

EEPROM_WriteByte(0x0123, 55);

Then these bytes are sent:

| Step | Value |
| --- | --- |
| Device Address | 0xA0 |
| First Word Address | 0x01 |
| Second Word Address | 0x23 |
| Data | 55 |

This matches the code:

buffer[0] = address >> 8;

If:

address = 0x0123

Then:

0x0123 >> 8

=

0x01

So:

buffer[0] = 0x01

Then:

buffer[1] = address & 0xFF;

0x0123 & 0xFF

=

0x23

So:

buffer[1] = 0x23

Then:

buffer[2] = data;

55

So the transmit buffer becomes:

buffer[0] = 0x01

buffer[1] = 0x23

buffer[2] = 55

The function prototype is approximately:

HAL_StatusTypeDef HAL_I2C_Master_Transmit(

I2C_HandleTypeDef *hi2c,

uint16_t DevAddress,

uint8_t *pData,

uint16_t Size,

uint32_t Timeout

);

Now let's examine your call:

HAL_I2C_Master_Transmit(

&hi2c1,

EEPROM_ADDR,

buffer,

3,

HAL_MAX_DELAY);

# Parameter 1: &hi2c1

&hi2c1

This tells the HAL:

"Use the I2C1 peripheral."

Remember:

I2C_HandleTypeDef hi2c1;

is created by CubeMX.

It contains information such as:

- I2C Instance (I2C1)

- Clock Speed

- Addressing Mode

- Current State

- Error Status

The & means address of.

The HAL function expects a pointer to this structure.

Without &, you'd pass the whole structure by value, but the function is defined to receive a pointer.

# Parameter 2: EEPROM_ADDR

EEPROM_ADDR

Suppose:

#define EEPROM_ADDR (0x50 << 1)

which evaluates to:

0xA0

This tells the HAL:

"Communicate with the EEPROM whose I²C address is 0xA0."

If you had an LCD I²C backpack:

Address = 0x4E

you would pass:

0x4E

instead.

# Parameter 3: buffer

Suppose:

uint8_t buffer[3];

buffer[0] = 0x01;

buffer[1] = 0x23;

buffer[2] = 55;

Memory looks like this:

Address      Value

--------     -----

0x20000000   0x01

0x20000001   0x23

0x20000002   55

When you write:

buffer

in C, it automatically becomes a pointer to the first element:

buffer

│

▼

0x20000000

So the HAL knows where your data starts.

# Parameter 4: 3

3

This tells HAL:

"Transmit three bytes."

Those three bytes are:

buffer[0]

buffer[1]

buffer[2]

If you wrote:

5

HAL would try to send:

buffer[0]

buffer[1]

buffer[2]

buffer[3]

buffer[4]

which would be incorrect because your array only contains three valid bytes.

# Parameter 5: HAL_MAX_DELAY

HAL_MAX_DELAY

This is the timeout.

Imagine the EEPROM never responds.

Without a timeout:

MCU

Wait...

Wait...

Wait...

Forever...

The program would hang forever.

Instead, HAL waits until either:

- Transmission completes ✅

- Timeout expires ❌

HAL_MAX_DELAY is the maximum timeout value, meaning:

"Wait as long as necessary."

# What actually happens on the I²C bus?

Suppose:

buffer[0] = 0x01;

buffer[1] = 0x23;

buffer[2] = 55;

The function call:

HAL_I2C_Master_Transmit(

&hi2c1,

EEPROM_ADDR,

buffer,

3,

HAL_MAX_DELAY);

results in this sequence:

STM32                           EEPROM

--------------------------------------------

START

-------->

Device Address (0xA0)

-------->

ACK

<--------

0x01

-------->

ACK

<--------

0x23

-------->

ACK

<--------

55

-------->

ACK

<--------

STOP

-------->

Notice something important:

The device address (0xA0) is NOT inside the buffer.

The HAL sends it automatically using the second parameter (EEPROM_ADDR).

Then it sends the three bytes stored in buffer.

# Why is buffer needed?

Imagine we write:

EEPROM_WriteByte(0x0123,55);

Inside the function:

buffer[0] = 0x01;

buffer[1] = 0x23;

buffer[2] = 55;

The buffer is simply a temporary array that holds the exact bytes we want to transmit after the device address.

# Interview Question

Interviewer: Explain each parameter of HAL_I2C_Master_Transmit().

A good answer is:

- &hi2c1 – Pointer to the initialized I²C peripheral handle.

- EEPROM_ADDR – I²C slave address of the EEPROM.

- buffer – Pointer to the data to be transmitted.

- 3 – Number of bytes to transmit.

- HAL_MAX_DELAY – Maximum time the HAL waits for the transfer to complete before timing out.

If you can also explain the I²C transaction (START → Device Address → Data Bytes → STOP), it demonstrates that you understand both the HAL API and what is happening on the bus underneath.

# What actually happens on the I²C bus?

Suppose:

buffer[0] = 0x01;

buffer[1] = 0x23;

buffer[2] = 55;

The function call:

HAL_I2C_Master_Transmit(

&hi2c1,

EEPROM_ADDR,

buffer,

3,

HAL_MAX_DELAY);

results in this sequence:

STM32                           EEPROM

--------------------------------------------

START

-------->

Device Address (0xA0)

-------->

ACK

<--------

0x01

-------->

ACK

<--------

0x23

-------->

ACK

<--------

55

-------->

ACK

<--------

STOP

-------->

Notice something important:

The device address (0xA0) is NOT inside the buffer.

The HAL sends it automatically using the second parameter (EEPROM_ADDR).

Then it sends the three bytes stored in buffer.

# Why is buffer needed?

Imagine we write:

EEPROM_WriteByte(0x0123,55);

Inside the function:

buffer[0] = 0x01;

buffer[1] = 0x23;

buffer[2] = 55;

The buffer is simply a temporary array that holds the exact bytes we want to transmit after the device address.

# Interview Question

Interviewer: Explain each parameter of HAL_I2C_Master_Transmit().

A good answer is:

- &hi2c1 – Pointer to the initialized I²C peripheral handle.

- EEPROM_ADDR – I²C slave address of the EEPROM.

- buffer – Pointer to the data to be transmitted.

- 3 – Number of bytes to transmit.

- HAL_MAX_DELAY – Maximum time the HAL waits for the transfer to complete before timing out.

If you can also explain the I²C transaction (START → Device Address → Data Bytes → STOP), it demonstrates that you understand both the HAL API and what is happening on the bus underneath.