# Project Title

Industrial Tank Level Monitoring System using STM32F091RC

## Objective

Design and develop an industrial-style tank level monitoring system that continuously measures liquid level, displays it on an LCD, stores configuration in EEPROM, generates alarms, and transmits real-time data to a PC through UART.

# Problem Statement

In industrial plants, storage tanks containing water or chemicals must be monitored continuously to prevent overflow and dry-run conditions.

Operators need a system that can:

- Measure liquid level in real time

- Display current level locally

- Generate alarms for abnormal conditions

- Log readings on a computer

- Retain alarm settings after power failure

The proposed system addresses these requirements using an STM32F091RC microcontroller.

| Component | Purpose |
| --- | --- |
| STM32F091RC NUCLEO | Main Controller |
| HC-SR04 Ultrasonic Sensor | Water level measurement |
| 16x2 LCD (PCF8574 I2C) | Local display |
| AT24C32 EEPROM | Store settings |
| Buzzer | Alarm |
| LEDs | Status indication |
| USB-UART | PC communication |
| Push Buttons | Menu/settings |

# Embedded Concepts Covered

This is where interviewers become interested.

### GPIO

LED

Buttons

Trigger pin

Echo pin

Buzzer

### Timer

Generate 10 µs trigger pulse

Measure Echo pulse width

Periodic sampling

### Interrupts

Optional button interrupt

UART interrupt

### UART

Transmit readings

Receive commands

### I2C

LCD

EEPROM

### EEPROM Driver

Read

Write

Page Write

Acknowledgement Polling

### State Machine

Normal

Low Alarm

High Alarm

Setting Mode

### Data Structures

Configuration structure

typedef struct

{

uint8_t lowAlarm;

uint8_t highAlarm;

uint16_t tankHeight;

}SystemConfig;

### Modular Programming

main.c

lcd.c

lcd.h

uart.c

uart.h

eeprom.c

eeprom.h

ultrasonic.c

ultrasonic.h

button.c

button.h

alarm.c

alarm.h

# Phase 1 – System Architecture

## Block Diagram

+----------------------+

|      STM32F091RC     |

|                      |

|  TIM2  ---> HC-SR04  |

|                      |

|  I2C1 ---> LCD       |

|         ---> EEPROM  |

|                      |

|  UART1 --> PC        |

|                      |

|  GPIO  --> LEDs      |

|         --> Buzzer   |

|         <-- Buttons  |

+----------------------+

# Software Architecture

Instead of writing everything in main.c, we'll divide the project into modules.

Core

│

├── main.c

│

├── ultrasonic.c

├── ultrasonic.h

│

├── lcd.c

├── lcd.h

│

├── uart.c

├── uart.h

│

├── eeprom.c

├── eeprom.h

│

├── alarm.c

├── alarm.h

│

├── button.c

├── button.h

│

├── config.c

├── config.h

│

└── tank.c

tank.h

This is exactly the kind of structure interviewers expect in professional firmware.

# Responsibilities of Each Module

## main.c

Only initializes peripherals and calls application functions.

while(1)

{

Tank_Update();

}

No sensor calculations or alarm logic here.

## ultrasonic.c

Responsible only for:

- Generate 10 µs trigger pulse

- Measure echo pulse

- Calculate distance

Provides:

float Ultrasonic_ReadDistance(void);

## tank.c

Contains application logic.

Responsibilities:

- Calculate level

- Calculate percentage

- Decide tank status

Example:

Level = TankHeight - Distance;

## alarm.c

Checks

Level < Low Alarm

or

Level > High Alarm

Turns

- LED

- Buzzer

ON/OFF.

## lcd.c

Displays

Tank Level

72%

or alarm messages.

## uart.c

Sends

Distance

Level

Percentage

Alarm Status

to PC.

## eeprom.c

Stores

Tank Height

Low Alarm

High Alarm

and reads them after reset.

## button.c

Reads buttons.

Provides

UP

DOWN

SET

functions.

## config.c

Stores all user settings.

Example

typedef struct

{

uint16_t tankHeight;

uint8_t lowAlarm;

uint8_t highAlarm;

}SystemConfig;

Only this module knows how configuration is saved and loaded.

# Recommended Pin Assignment

| Peripheral | STM32 Pin |
| --- | --- |
| HC-SR04 Trigger | PA0 |
| HC-SR04 Echo | PA1 (TIM2 Input Capture) |
| LCD SDA | PB7 |
| LCD SCL | PB6 |
| EEPROM SDA | PB7 (same I²C bus) |
| EEPROM SCL | PB6 |
| UART TX | PA9 |
| UART RX | PA10 |
| Buzzer | PA5 |
| Red LED | PA6 |
| Green LED | PA7 |
| Button SET | PB0 |
| Button UP | PB1 |
| Button DOWN | PB2 |

# Phase 2 – STM32CubeMX Configuration

## Step 1: Create a New Project

- Open STM32CubeIDE

- Click File → New → STM32 Project

- Search for NUCLEO-F091RC (or STM32F091RCT6 if using the MCU directly)

- Click Next

- Project Name:

Industrial_Tank_Level

Language:

C

Toolchain:

STM32CubeIDE

Click Finish.

# Step 2: Clock Configuration

For this project, 48 MHz is sufficient and is the maximum CPU frequency for the STM32F091RC.

Go to Clock Configuration.

Configure:

SYSCLK = 48 MHz

AHB = 48 MHz

APB1 = 48 MHz

This gives enough performance while keeping timer calculations simple.

### Why 48 MHz?

We need:

- Timer for ultrasonic pulse measurement

- UART communication

- I²C communication

- LCD updates

- EEPROM access

48 MHz easily handles all of these simultaneously.

Interview Question:

Why didn't you use a lower clock?

Answer:

Higher timer resolution improves distance measurement accuracy, while still remaining within the STM32F091RC's specifications.

# Step 3: GPIO Configuration

Configure the GPIO pins as follows:

| Pin | Function | Mode |
| --- | --- | --- |
| PA0 | HC-SR04 Trigger | GPIO Output |
| PA1 | HC-SR04 Echo | TIM2 Input Capture |
| PA5 | Buzzer | GPIO Output |
| PA6 | Red LED | GPIO Output |
| PA7 | Green LED | GPIO Output |
| PB0 | SET Button | GPIO Input + Pull-up |
| PB1 | UP Button | GPIO Input + Pull-up |
| PB2 | DOWN Button | GPIO Input + Pull-up |

### Why Pull-up?

The input stays at logic '1' normally. Pressing the button connects it to GND, producing a clean logic '0' without a floating input.

# Step 4: Configure I²C1

Enable I²C1.

Use:

| Pin | Function |
| --- | --- |
| PB6 | SCL |
| PB7 | SDA |

Speed:

100 kHz

### Why only one I²C bus?

Both devices have different addresses:

- LCD (PCF8574): for example 0x27

- EEPROM (AT24C32): typically 0x50 (7-bit address)

Both can share the same SDA and SCL lines.

Interview Question:

How can two devices use the same wires?

Answer:

The master addresses each device individually. Only the addressed slave responds.

# Step 5: Configure UART1

Enable USART1 in Asynchronous mode.

Pins:

| Pin | Function |
| --- | --- |
| PA9 | TX |
| PA10 | RX |

Settings:

Baud Rate = 115200

Data Bits = 8

Parity = None

Stop Bits = 1

### Why 115200?

Fast enough for continuous logging without stressing the CPU.

Example output:

Distance: 28 cm

Level: 72 cm

Percentage: 72%

Alarm: NORMAL

# Step 6: Configure TIM2

TIM2 will perform two jobs:

- Generate the 10 µs trigger pulse (software-assisted)

- Measure the echo pulse width using Input Capture

Enable:

TIM2

Select:

Channel 2 → Input Capture Direct Mode

Assign the echo pin (e.g., PA1) to TIM2 Channel 2 if supported by your package.

### Why Input Capture?

Without Input Capture:

- CPU continuously polls the pin.

- Lower accuracy.

With Input Capture:

- Timer automatically captures the count value on signal edges.

- High precision with minimal CPU involvement.

This is the professional approach.

# Understanding the HC-SR04 Before Writing Any Code

Rule: A good embedded engineer understands the hardware first, then writes the firmware.

# What is the HC-SR04?

It is an ultrasonic distance sensor.

It measures distance using sound waves, not light.

It has 4 pins:

HC-SR04

+------------------+

|                  |

| VCC   TRIG  ECHO GND

+------------------+

| Pin | Function |
| --- | --- |
| VCC | +5V Supply |
| TRIG | Input from MCU |
| ECHO | Output to MCU |
| GND | Ground |

# How Does It Work?

The STM32 sends a 10 µs pulse to the Trigger pin.

Trigger

_____|‾‾‾‾|_________________

10 µs

The HC-SR04 then sends 8 ultrasonic pulses at 40 kHz.

If those sound waves hit an object, they bounce back.

The sensor measures the round-trip time and outputs it as the width of the Echo pulse.

Echo

___________|‾‾‾‾‾‾‾‾‾‾‾‾‾|__________

Pulse Width

The farther the object, the longer the pulse.

# Internal Operation

STM32

│

│ 10 µs Trigger

▼

HC-SR04

│

│ Sends Ultrasound

▼

Object

│

│ Reflects Sound

▼

HC-SR04

│

│ Echo Pulse

▼

STM32 Timer

# Why Measure Pulse Width?

Suppose:

Object is 34 cm away.

Sound travels:

Sensor → Object → Sensor

So total distance:

34 + 34 = 68 cm

Speed of sound:

343 m/s

Convert to cm/µs:

34300 cm/s

34300 / 1,000,000

0.0343 cm/µs

Time:

68 / 0.0343

≈1982 µs

The Echo pulse width will be about 1982 µs.

# Distance Formula

Since the sound travels to the object and back:

Distance=Time×0.03432\text{Distance}=\frac{\text{Time}\times0.0343}{2}Distance=2Time×0.0343​

In C:

distance = (pulse_width * 0.0343f) / 2.0f;

Many examples simplify it to:

distance = pulse_width / 58.0f;

Both formulas produce nearly the same result.

# Why Use a Timer?

Imagine trying to count:

1982 µs

using software loops.

Problems:

- CPU speed affects accuracy

- Compiler optimization changes timing

- Interrupts introduce errors

Instead:

Echo

│

▼

Timer Input Capture

│

▼

Captured Count

│

▼

Pulse Width

The timer counts in hardware, making the measurement accurate and independent of CPU execution speed.

# Which Edge Does the Timer Capture?

The Echo pulse has two important edges.

Echo

_____|‾‾‾‾‾‾‾‾|_______

↑       ↑

Rising   Falling

We need both.

- Rising edge → Start time

- Falling edge → End time

Then:

Pulse Width

= End − Start

# Example

Suppose:

Rising edge captured:

1500

Falling edge captured:

3482

Then:

Pulse Width

3482 - 1500

=1982

Distance:

1982 / 58

≈34 cm

# Interview Questions

### Q1. Why is the distance divided by 2?

Because the ultrasonic wave travels:

- Sensor → Object

- Object → Sensor

The measured time is for the round trip, so we divide by 2 to obtain the one-way distance.

### Q2. Why is Input Capture preferred over GPIO polling?

Because Input Capture uses hardware to latch the timer value on signal edges, providing high timing accuracy with very little CPU involvement. Polling consumes CPU time and is less precise.

### Q3. Why do we need both rising and falling edges?

The pulse width equals:

Falling Time − Rising Time

Capturing only one edge does not provide the pulse duration.