Step-by-step debugging is exactly what I recommend here. We want to trace:

main.c → EEPROM_WriteByte() → HAL_I2C_Master_Transmit() → status → Error_Handler()

### 1. Put a breakpoint here

In eeprom.c:

status = HAL_I2C_Master_Transmit(&hi2c1,

EEPROM_ADDR,

buffer,

3,

HAL_MAX_DELAY);

Put a breakpoint on this line.

Then start debugging.

### 2. When execution stops there

Before executing the line, inspect:

EEPROM_ADDR

buffer[0]

buffer[1]

buffer[2]

For your test:

EEPROM_WriteByte(0x0000, 25);

you should see:

EEPROM_ADDR = 0xA0

buffer[0]   = 0x00

buffer[1]   = 0x00

buffer[2]   = 0x19

because 25 decimal = 0x19.

### 3. Press F6 / Step Over

Execute:

status = HAL_I2C_Master_Transmit(...);

Now look at:

status

The important possibilities are:

HAL_OK

HAL_ERROR

HAL_BUSY

HAL_TIMEOUT

### 4. If you get HAL_ERROR

The next line is:

if (status != HAL_OK)

{

Error_Handler();

}

Press F6 (Step Over).

You should then enter:

void Error_Handler(void)

{

__disable_irq();

while (1)

{

}

}

The debugger will stop inside the infinite:

while (1)

That proves:

HAL_I2C_Master_Transmit() failed.

### Your complete debugging path

main.c

│

▼

EEPROM_WriteByte(0x0000, 25)

│

▼

eeprom.c

│

▼

HAL_I2C_Master_Transmit()

│

├── HAL_OK ────────► continue

│

└── HAL_ERROR ────► Error_Handler()

│

▼

while(1)

i m getting HAL_ERROR

Good. This is useful information. We have now isolated the problem:

HAL_I2C_Master_Transmit() is failing, so the STM32 is not successfully communicating with the EEPROM.

Your menu code is not the problem yet.

Let's debug the I²C communication systematically.

### Step 1 — Check the actual I²C error code

Immediately after:

status = HAL_I2C_Master_Transmit(&hi2c1,

EEPROM_ADDR,

buffer,

3,

HAL_MAX_DELAY);

look at:

hi2c1.ErrorCode

In the debugger, inspect:

hi2c1.ErrorCode

Tell me what value you get.

You may see something such as:

HAL_I2C_ERROR_AF

HAL_I2C_ERROR_BERR

HAL_I2C_ERROR_ARLO

HAL_I2C_ERROR_TIMEOUT

The exact error code is important because it tells us what happened on the I²C bus.

hi2c1.ErrorCode is not a separate variable you created. It is a member inside the STM32 HAL I²C handle structure.

Let's find it step-by-step in STM32CubeIDE.

### Method 1 — Use Expressions window

- Start debugging.

- Stop at this line in eeprom.c:

status = HAL_I2C_Master_Transmit(&hi2c1,

EEPROM_ADDR,

buffer,

3,

HAL_MAX_DELAY);

- In the debugger, look for the Expressions window.

If you don't see it:

Window → Show View → Expressions

- Click Add new expression (+).

- Enter:

hi2c1.ErrorCode

- Press Enter.

You should then see something like:

hi2c1.ErrorCode    0x00000004

### Step 2 — Also check these values

At the same breakpoint, verify:

EEPROM_ADDR = 0xA0

buffer[0]   = 0x00

buffer[1]   = 0x00

buffer[2]   = 0x19

If these are correct, don't change the EEPROM driver yet.

i have connected eeprom and lcd in paralled ,lcd is working so sda n scl no problem i guess

Yes — that is a strong clue. If your LCD is connected to the same I²C SDA/SCL lines and is working, then:

- STM32 I²C peripheral is working.

- SDA/SCL are probably wired correctly.

- I²C clock configuration is probably working.

- The bus itself is functioning.

But it does not prove the EEPROM is reachable, because the LCD and EEPROM have different I²C addresses.

Your HAL_I2C_ERROR_AF (4) means the EEPROM address you are transmitting to is not being acknowledged.

### Let's test the EEPROM address directly

You currently have:

#define EEPROM_ADDR 0xA0

Let's see whether the STM32 gets an ACK from that address.

Temporarily add this after MX_I2C1_Init();:

HAL_StatusTypeDef eepromStatus;

eepromStatus = HAL_I2C_IsDeviceReady(

&hi2c1,

EEPROM_ADDR,

3,

100);

Put a breakpoint after this line:

eepromStatus = HAL_I2C_IsDeviceReady(...);

Then inspect:

eepromStatus

### Expected result

If EEPROM is detected:

eepromStatus = HAL_OK

If it isn't detected:

eepromStatus = HAL_ERROR

This test is better than trying to write/read because it answers one very specific question:

"Does anything at address 0xA0 acknowledge the STM32?"

hal_error

Good. This confirms something important:

HAL_I2C_IsDeviceReady() = HAL_ERROR for 0xA0 means the EEPROM is not acknowledging address 0xA0.

Since your LCD works on the same SDA/SCL bus, let's not touch the STM32 I²C configuration or your EEPROM read/write code yet.

### Next: find the EEPROM's actual I²C address

The cleanest test is to scan the I²C bus.

Temporarily put this in main.c, after:

MX_I2C1_Init();

inside USER CODE BEGIN 2:

uint8_t address;

for (address = 1; address < 128; address++)

{

if (HAL_I2C_IsDeviceReady(&hi2c1,

address << 1,

2,

10) == HAL_OK)

{

// Put a breakpoint here

// address contains the detected 7-bit I2C address

}

}

### What to do in the debugger

Put a breakpoint on:

if (HAL_I2C_IsDeviceReady(&hi2c1,

Then use Step Over (F6) repeatedly.

When the condition becomes true, inspect:

address

You should find your LCD and EEPROM addresses.

For example, you might get:

address = 0x27

for the LCD, and hopefully:

address = 0x50

for the AT24C32.

Remember:

7-bit address = 0x50

becomes:

0x50 << 1 = 0xA0

for STM32 HAL.

You don't need to write anything there. That blank block was only meant to give you a place to put a breakpoint.

But to make it easier to debug, I recommend writing a variable so you can clearly see the detected address.

Use this:

uint8_t address;

uint8_t detectedAddress = 0;

for (address = 1; address < 128; address++)

{

if (HAL_I2C_IsDeviceReady(&hi2c1,

address << 1,

2,

10) == HAL_OK)

{

detectedAddress = address;

// Put breakpoint on the next line

}

}

### What happens

The STM32 checks:

0x01

0x02

0x03

...

0x27

...

0x50

...

0x7F

When a device responds, this condition becomes true:

if (HAL_I2C_IsDeviceReady(...) == HAL_OK)

Then:

detectedAddress = address;

stores the detected 7-bit address.

### Important problem with this version

If you have both LCD and EEPROM connected, there will be more than one detected address.

For example:

LCD     → 0x27

EEPROM  → 0x50

So detectedAddress will eventually contain the last detected device.

A better debugging version is:

uint8_t address;

for (address = 1; address < 128; address++)

{

if (HAL_I2C_IsDeviceReady(&hi2c1,

address << 1,

2,

10) == HAL_OK)

{

__NOP();   // Put breakpoint here

}

}

### What you should do

Put a breakpoint on:

__NOP();

Then press Resume (F8).

Every time the STM32 finds an I²C device, it will stop at __NOP().

Look at:

address

If you get:

address = 0x27

that's probably your LCD.

Then press Resume again.

If you get:

address = 0x50

🎯 That's your AT24C32.

Then your EEPROM address should be:

#define EEPROM_ADDR 0xA0

because:

0x50 << 1 = 0xA0

This scan is the best next test because we can determine the actual address of your EEPROM instead of guessing.

That's okay. The likely reason is that __NOP() is not a good breakpoint target in this case, especially with compiler optimization. Let's make the test much easier to trace.

### Do this instead

Temporarily replace the scan with:

uint8_t address;

HAL_StatusTypeDef testStatus;

for (address = 1; address < 128; address++)

{

testStatus = HAL_I2C_IsDeviceReady(&hi2c1,

address << 1,

2,

10);

if (testStatus == HAL_OK)

{

volatile uint8_t foundAddress = address;

while (1)

{

}

}

}

Now put a breakpoint on:

volatile uint8_t foundAddress = address;

### Why this is better

When an I²C device responds:

HAL_I2C_IsDeviceReady()

↓

HAL_OK

↓

foundAddress = address

↓

STOP

The program will stay in the while(1) after finding the first device, so you won't miss the breakpoint.

### But there is an even easier method

Since your LCD is already working, let's test the EEPROM directly instead of scanning the entire bus.

Use:

HAL_StatusTypeDef eepromStatus;

eepromStatus = HAL_I2C_IsDeviceReady(

&hi2c1,

0xA0,

10,

100);

Put a breakpoint on:

eepromStatus = HAL_I2C_IsDeviceReady(

Then:

- Start debugging.

- When breakpoint hits, press F6 (Step Over).

- Look at eepromStatus.

You already did this and got HAL_ERROR, so we know 0xA0 isn't responding.

If you mean address = 39 from the I²C scan, that's actually useful.

39 decimal = 0x27 hexadecimal.

So your scan found:

address = 39 decimal

= 0x27

That is very likely your LCD's I²C address.

### This tells us

Your scan is working because it found the LCD:

STM32

│

└── I²C bus

└── LCD → 0x27 ✅

Now we need to see whether the scan also finds the EEPROM.

Your loop should continue scanning after finding 39. Don't put while(1) inside the first detection, because that stops the scan at the LCD and never allows it to reach the EEPROM.

Use this:

uint8_t address;

volatile uint8_t foundAddress;

for (address = 1; address < 128; address++)

{

if (HAL_I2C_IsDeviceReady(&hi2c1,

address << 1,

2,

10) == HAL_OK)

{

foundAddress = address;

HAL_Delay(10);

}

}

Put a breakpoint on:

foundAddress = address;

Then:

- Run/resume.

- It should stop at 39.

- Press Resume (F8).

- It should stop again if another I²C device is detected.

- Check foundAddress each time.

### What we're looking for

Ideally you'll get:

39 decimal = 0x27 → LCD

80 decimal = 0x50 → AT24C32

If you get only 39, then:

LCD     → detected ✅

EEPROM  → not detected ❌

And that would explain your HAL_ERROR / ErrorCode = 4.

it get hang forever

Yes — that behavior makes sense. Stop using that scan for now. I gave you a debugging method that can become awkward inside a live HAL/I²C transaction.

We already learned something valuable:

- LCD responds at 39 decimal = 0x27 ✅

- EEPROM at 0xA0 gives ACK failure (ErrorCode = 4) ❌

So let's isolate the EEPROM without scanning.

### Do this simple test

Temporarily remove/comment the scan code.

After:

MX_I2C1_Init();

put:

HAL_StatusTypeDef eepromStatus;

eepromStatus = HAL_I2C_IsDeviceReady(&hi2c1,

0xA0,

3,

100);

Put a breakpoint on the next line:

eepromStatus = HAL_I2C_IsDeviceReady(&hi2c1,

0xA0,

3,

100);

__NOP();   // breakpoint here

Then:

- Start debugging.

- Let it reach __NOP().

- Inspect eepromStatus.

You already got HAL_ERROR, so I expect the same result.

## Now check the EEPROM address pins physically

This is the important part.

For an AT24C32:

A2 A1 A0

0  0  0  → 7-bit address 0x50

STM32 HAL:

0x50 << 1 = 0xA0

Therefore your:

#define EEPROM_ADDR 0xA0

is correct only if A0, A1 and A2 are all LOW.

### Please check these three pins

A0 → GND

A1 → GND

A2 → GND

Also:

WP → GND

### One very important possibility

If you're using an AT24C32 module, don't assume the address pins are grounded just because you didn't connect them. Some modules have solder bridges/jumpers that determine A0/A1/A2.

And if your EEPROM is actually a different chip/module, its address configuration may be different.

Send me a clear photo of the EEPROM you connected (including the labels/pins). I can then tell you exactly what address it should have and how each pin should be connected.

TO CHECK WHICH DEVICES CONNECTED ON I2C LINE USE THIS

uint8_t found = 0;

for(uint8_t address = 1; address < 128; address++)

{

if(HAL_I2C_IsDeviceReady(&hi2c1,

address << 1,

2,

10) == HAL_OK)

{

found = address;

}

}

Put breakpoint at found line and see address in variables