# Lesson 10: Display Distance and Water Level on LCD

## Goal

We want the LCD to show:

Level: 64%

Dist : 18.3cm

This updates every time a new measurement is taken.

# Step 1: Include Required Header Files

In main.c, add:

#include "ultrasonic.h"

#include "i2c_lcd.h"

#include <stdio.h>

### Why <stdio.h>?

Because we'll use:

sprintf();

to convert numbers into strings.

# Step 2: Create LCD Buffers

Near the top of main.c, inside the USER CODE variable section:

char lcdLine1[17];

char lcdLine2[17];

### Why 17?

A 16×2 LCD displays 16 characters per line.

We need one extra byte for the string terminator ('\0').

So:

16 characters

+

1 null character

=

17 bytes

# Step 3: Format the First Line

Suppose:

Level = 64%

Write:

sprintf(lcdLine1, "Level:%3.0f%%", levelPercent);

Let's understand this format string.

### %3.0f

- % → format specifier

- 3 → minimum width of 3 characters

- .0 → no digits after the decimal point

- f → floating-point number

If:

levelPercent = 64.2

Output becomes:

Level: 64%

Notice the %%.

Why?

Because % has a special meaning in sprintf().

To print a literal %, you write:

%%

# Step 4: Format the Second Line

sprintf(lcdLine2, "Dist:%5.1fcm", distance);

Suppose:

distance = 18.34

Output:

Dist: 18.3cm

### %5.1f

- Width = 5

- One digit after the decimal

Examples:

5.2

18.3

123.4

All align nicely on the LCD.

# Step 5: Display on LCD

lcd_put_cur(0,0);

lcd_send_string(lcdLine1);

lcd_put_cur(1,0);

lcd_send_string(lcdLine2);

Meaning:

Row 0 Column 0

↓

Level:64%

and

Row 1 Column 0

↓

Dist:18.3cm

# Step 6: Where to Place This Code?

Inside your while(1) loop:

while (1)

{

HCSR04_Trigger();

HAL_Delay(100);

float distance = HCSR04_GetDistance();

float waterLevel = TANK_HEIGHT_CM - distance;

float levelPercent =

(waterLevel * 100.0f) / TANK_HEIGHT_CM;

sprintf(lcdLine1,

"Level:%3.0f%%",

levelPercent);

sprintf(lcdLine2,

"Dist:%5.1fcm",

distance);

lcd_put_cur(0,0);

lcd_send_string(lcdLine1);

lcd_put_cur(1,0);

lcd_send_string(lcdLine2);

}

# How the Data Flows

HC-SR04

│

▼

Distance

│

▼

Water Level

│

▼

Percentage

│

▼

sprintf()

│

▼

Character String

│

▼

LCD

Notice something important:

The LCD cannot display a float directly.

It only understands characters.

That's why:

64.2

must first become:

"64.2"

using sprintf().

# Interview Question

### Why do we use sprintf()?

Answer:

The LCD driver accepts character strings, not floating-point numbers. sprintf() converts numerical values into formatted ASCII strings that can be displayed on the LCD.

# One Optimization

Many beginners do this every loop:

lcd_clear();

Then print the new values.

Don't do that.

lcd_clear();

causes the LCD to blink because clearing the display is relatively slow.

A better approach is exactly what we're doing:

lcd_put_cur(...);

lcd_send_string(...);

Overwrite the previous text without clearing the display.