Excellent. Now we are going to build one of the most industry-relevant parts of the project.

Until now we have:

- ✅ Ultrasonic sensor

- ✅ LCD

- ✅ LEDs

- ✅ Buzzer

- ✅ EEPROM Driver

Now we'll combine them into a real embedded application.

# Lesson 15: Settings Menu using SET, UP and DOWN Buttons

## Why do we need buttons?

Imagine this tank is installed in a factory.

Today the manager wants:

Low Level Alarm = 20%

After one month:

A different tank is installed.

Now he wants:

Low Level Alarm = 35%

Should he:

- Open STM32CubeIDE?

- Change the code?

- Compile?

- Flash the MCU?

❌ Absolutely not.

The operator should be able to change the settings from the device itself.

# Our Menu

We have three buttons.

SET

UP

DOWN

They will work like this:

Press SET

↓

Enter Settings Mode

LCD:

LOW LEVEL

20%

Press UP

21%

Press UP

22%

Press DOWN

21%

Press SET again

Saved to EEPROM

Exit settings mode.

# State Diagram

Every embedded engineer should think in terms of states.

SET

Normal -----------------> Settings

SET

Settings -----------------> Normal

There are only two states.

# Step 1: Create a State Variable

At the top of main.c, inside:

/* USER CODE BEGIN PV */

add:

uint8_t settingMode = 0;

### What does it mean?

When:

settingMode = 0;

the system is in:

NORMAL MODE

When:

settingMode = 1;

the system is in:

SETTINGS MODE

# Why use uint8_t?

Because we only need:

0

or

1

One byte is enough.

# Step 2: Threshold Variable

Earlier we had

#define LOW_LEVEL_THRESHOLD 20

Remove it.

Instead create

uint8_t LowLevelThreshold;

Now the threshold can change while the program is running.

# Step 3: Initialize the Threshold

Inside main() after all peripheral initialization:

LowLevelThreshold = EEPROM_ReadByte(0x0000);

But what if the EEPROM is brand new?

A new EEPROM contains:

0xFF

in every location.

If we directly read:

255

then:

Threshold = 255%

which makes no sense.

So we check:

if(LowLevelThreshold == 0xFF)

{

LowLevelThreshold = 20;

}

This gives a sensible default on the first power-up.

# Step 4: Detect SET Button

In your while(1) loop:

if(HAL_GPIO_ReadPin(SET_BUTTON_GPIO_Port,

SET_BUTTON_Pin)==GPIO_PIN_RESET)

{

HAL_Delay(200);

settingMode = !settingMode;

}

## Why !settingMode?

This is a neat C trick.

Suppose:

settingMode = 0

Then:

!settingMode

becomes:

1

Suppose:

settingMode = 1

Then:

!settingMode

becomes:

0

So every time SET is pressed:

0 → 1

1 → 0

0 → 1

1 → 0

We switch between Normal Mode and Settings Mode.

# Step 5: Different Behavior in Each State

Instead of writing everything directly inside while(1), use:

if(settingMode == 0)

{

// Normal operation

// Measure tank level

// Display water level

// Alarm

}

else

{

// Settings Mode

}

This makes the code much easier to understand and maintain.

# Interview Tip

If an interviewer asks:

Why did you use a state variable?

You can answer:

"The firmware has two operating modes: Normal Mode and Settings Mode. Instead of writing complicated nested conditions, I used a state variable to control the system behavior. This approach is scalable, readable, and is a common finite state machine (FSM) design pattern used in embedded systems."

## Your Task Before We Continue

Implement only these steps:

- Create settingMode

- Create LowLevelThreshold

- Read the threshold from EEPROM

- Apply the default value if EEPROM contains 0xFF

- Toggle settingMode when the SET button is pressed

- Create the two-mode structure:

if(settingMode == 0)

{

// Normal mode

}

else

{

// Settings mode

}

Don't implement the UP/DOWN buttons yet.

Once this structure is working, we'll make the UP and DOWN buttons modify the threshold and then save it back to EEPROM when the user exits Settings Mode. This is the point where your firmware starts looking like a real industrial embedded application rather than a collection of peripheral examples.

Excellent. Now your project has two modes:

- Normal Mode (settingMode = 0)

- Settings Mode (settingMode = 1)

Now we'll make the UP and DOWN buttons actually change the threshold.

# Step 16: Change Threshold Using UP and DOWN Buttons

## Goal

When in Settings Mode:

- UP → Increase threshold

- DOWN → Decrease threshold

- LCD shows the updated value immediately

Example:

LOW LEVEL

20%

Press UP

LOW LEVEL

21%

Press UP

LOW LEVEL

22%

Press DOWN

LOW LEVEL

21%

# Step 1: Only Work in Settings Mode

Inside your while(1) you already have:

if(settingMode == 0)

{

// Normal mode

}

else

{

// Settings mode

}

We will write the button code inside the else block only.

# Step 2: Read the UP Button

Inside the else block:

if(HAL_GPIO_ReadPin(UP_BUTTON_GPIO_Port,

UP_BUTTON_Pin) == GPIO_PIN_RESET)

{

HAL_Delay(200);

LowLevelThreshold++;

}

### What happens?

Suppose:

LowLevelThreshold = 20

Press UP:

20

↓

21

Press again:

21

↓

22

# Step 3: Read the DOWN Button

Below the UP button code:

if(HAL_GPIO_ReadPin(DOWN_BUTTON_GPIO_Port,

DOWN_BUTTON_Pin) == GPIO_PIN_RESET)

{

HAL_Delay(200);

LowLevelThreshold--;

}

Example:

22

↓

21

↓

20

↓

19

# Step 4: Prevent Invalid Values

Suppose someone keeps pressing UP.

Eventually:

100

101

102

150

But a tank level percentage should only be 0–100%.

So modify the UP code:

if(HAL_GPIO_ReadPin(UP_BUTTON_GPIO_Port,

UP_BUTTON_Pin) == GPIO_PIN_RESET)

{

HAL_Delay(200);

if(LowLevelThreshold < 100)

{

LowLevelThreshold++;

}

}

Similarly for DOWN:

if(HAL_GPIO_ReadPin(DOWN_BUTTON_GPIO_Port,

DOWN_BUTTON_Pin) == GPIO_PIN_RESET)

{

HAL_Delay(200);

if(LowLevelThreshold > 0)

{

LowLevelThreshold--;

}

}

Now:

- UP stops at 100

- DOWN stops at 0

# Step 5: Display the Threshold

Instead of showing distance and water level, the LCD should show the setting being edited.

Inside the else block:

sprintf(lcdLine1, "LOW LEVEL");

sprintf(lcdLine2, "%3d%%", LowLevelThreshold);

lcd_put_cursor(0, 0);

lcd_send_string("                ");   // Clear line

lcd_put_cursor(0, 0);

lcd_send_string(lcdLine1);

lcd_put_cursor(1, 0);

lcd_send_string("                ");   // Clear line

lcd_put_cursor(1, 0);

lcd_send_string(lcdLine2);

Now the LCD becomes:

LOW LEVEL

20%

Press UP:

LOW LEVEL

21%

# Why Clear the LCD Line?

Suppose the LCD previously displayed:

LOW LEVEL

100%

Then the value becomes:

99%

Without clearing, the LCD might show:

99%%

or leave old characters behind.

Writing spaces first ensures the old text is removed before displaying the new value.

# Test This Before Moving On

When you press SET:

Normal Mode

↓

Settings Mode

The LCD should display:

LOW LEVEL

20%

Press:

- UP → 21, 22, 23...

- DOWN → 22, 21, 20...

- Never go below 0

- Never go above 100